| Registration No.: | N | Start |  | Aircraft Start Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Start Total Cycles: |  |

Reference:  AW119MKII Maintenance Planning Manual Chapter 05-50

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - GENERAL |  |  |
| - Logbook review for recorded discrepancies. Correct discrepancies on installed equipment. |  |  |
| - Ensure all applicable and required Special Inspections, Alert Service Bulletins (ASB’s), Engine Service Bulletins, and Airworthiness Directives (AD’s) are accomplished. |  |  |
| - Replace all life limited components that have completed published operating limits. | AW119MKII MPM |  |
| - Overhaul all components that have completed published operating limits. | AW119MKII MPM |  |
| - PLACARDS AND MARKINGS |  |  |
| - Examine the placards, decals, and markings. Make sure you can read them, they are applied correctly, and they are in agreement with the applicable configuration of your helicopter. | Chp 11 |  |
| - FUSELAGE UPPER DECK |  |  |
| - Transmission support assemblies and relative fittings secured to fuselage. | 63-31-5 |  |
| - MAIN ROTOR |  |  |
| - Grips, pitch control levers, flap and droop restrainers for condition, damage and wear (hub disassembly required). | 62-21-13.23 62-21-33,35,55 |  |
|  | 35,39,55 |  |
| - Inspect blades grips bushings (blade retention pin seat) for wear, nicks and condition. | 62-21-23 |  |
| - Inspect contact area between blade and grip for condition and washer’s bondage. |  |  |
| - Main rotor head installation. | 62-21-14 |  |
| - Hub floating ring for wear and DU washers and internal teflon shim for debonding. | 62-21-44 |  |
| - Inspect blade root bushings and blade root washers for damage and wear. | 62-11-25,26 |  |
|  | 62A-11-25,26 |  |
| - Main rotor pitch change link assembly. | 62-31-10 |  |
| - Inspect main rotor grip droop stop plate for wear and damage. | 62-21-23 |  |
|  | Step F |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - MAIN ROTOR CONTROLS |  |  |
| - Remove and inspect swashplate duplex bearing for damage and corrosion. | 62-31-25 |  |
| - Swashplate teflon rings (pivot sleeve and swashplate) for wear. | 62-31-25 |  |
| - Guide and pivot sleeve slot of swashplate for damage and wear. | 62-31-25 |  |
| - POST INSPECTION |  |  |
| - Replace or close all applicable trim panels, covers, access and inspection doors/covers as necessary. |  |  |
| - Complete a ground run at 100% NR to check for leaks and confirm system operation. |  |  |
| - Perform Functional Check Flight if required per AW119MKII MPM Maintenance Manual or RFM as appropriate. |  |  |
| - Make an entry in the appropriate logbook (refer to the following page for wording of approval for return to service). |  |  |

Upon satisfactory completion of each inspection item, the mechanic shall initial the pertinent space as provided.  All mechanics shall sign and initial, in the space provided, at the end of the form.

At the completion of each inspection form, an A&P mechanic shall check the entire form for completion and then enter his/her signature, certificate type and number, and date in the space provided at the end of the form.  The same individual is also responsible for making the logbook entry shown at the conclusion of this inspection.

I certify that I have inspected and/or completed all the items initialed by me in accordance with Maritime Helicopters’ AW119MKII Approved Aircraft Inspection Program and found them to be in an airworthy condition except as may be noted in the Aircraft Logbook.

| INITIALS | SIGNATURE | CERTIFICATE TYPE & NUMBER |
| --- | --- | --- |

I have reviewed this inspection form to verify that all items have been completed and have made an entry in the Aircraft Log Book.

I certify that this aircraft has been inspected in accordance with Maritime Helicopters’ AW119MKII AAIP – A 2400RT – 2400 Hour Inspection and was determined to be in an airworthy condition.

| Registration No.: | N | End |  | Aircraft Ending Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Ending Total Cycles: |  |

| Signature: |  |  | Certificate Type & No: |  |
| --- | --- | --- | --- | --- |