| Registration No.: | N | Start |  | Aircraft Start Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Location: |  |

Reference:  Eurocopter B0-117 Series MM Appendix A

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - GENERAL |  |  |
| - Logbook review for recorded discrepancies. Correct discrepancies on installed equipment. |  |  |
| - Ensure all applicable and required Special Inspections, Alert Service Bulletins (ASB’s), Engine Service Bulletins, and Airworthiness Directives (AD’s) are accomplished. |  |  |
| - Replace all life limited components that have completed published operating limits. | Airbus |  |
|  | BK-117 MP |  |
| - Overhaul all components that have completed published operating limits. | Airbus |  |
|  | BK-117 MP |  |
| - PLACARDS AND MARKINGS |  |  |
| - Examine the placards, decals, and markings. Make sure you can read them, they are applied correctly, and they are in agreement with the applicable configuration of your helicopter. |  |  |
| - CORROSION CONTROL |  |  |
| - Perform corrosion inspection in any area’s access during this AAIP inspection. |  |  |
| - MAIN TRANSMISSION |  |  |
| - ROTOR BRAKE SYSTEM |  |  |
| - MAIN ROTOR SYSTEM |  |  |

| - MAIN ROTOR BLADE |  |  |
| --- | --- | --- |
| - Perform lead--lag check or alternatively inspect the blade thimble area. |  |  |
| Effectivity: Main rotor blades (blade sweep angle 1 degree) P/N 117--150001, P/N 117--150011 and all versions, 117--150021 and all versions, 117--50061 and all versions, 117--151321 and all versions, 117--151341 and all versions, 117--151351 and all versions, 117--151361 and all versions. |  |  |
| - FUSELAGE |  |  |
| - COWLINGS |  |  |
| - Check latch of engine cowlings and transmission cowlings for function | 22--10, 22--14 |  |
| EFFECTIVITY: After performance of SB--MBB--BK 117--20--109 |  |  |
| - TAIL BOOM |  |  |
| - TAIL ROTOR DRIVE |  |  |
| - TAIL ROTOR |  |  |
| - TAIL ROTOR BLADE |  |  |
| - MAIN ROTOR FLIGHT CONTROLS |  |  |
| - TAIL ROTOR FLIGHT CONTROLS |  |  |
| - HYDRAULIC SYSTEM |  |  |
| - LANDING GEAR |  |  |
| - FUEL SYSTEM |  |  |
| - Check fuel shutoff valves for function and evidence of leakage. | 62-39 step 5 |  |
| - Inspect fuel system for leakage. | 62-40 |  |
| - LUBRICATION SYSTEM |  |  |
| - FIRE WALLS |  |  |
| - ENGINE OPERATION AND CONTROL |  |  |
| - STANDARD EQUIPMENT |  |  |
| - INSTRUMENT SYSTEMS |  |  |
| - ELECTRICAL SYSTEMS |  |  |
| - Inspect ground leads and ground connections of fueling nozzle and fuselage/landing gear for condition. |  |  |
| - Inspect bonding connections and bonding wire for condition without disassembling. | 92--33 step 2 and 02--15 |  |
| - DC SUPPLY SYSTEM |  |  |
| - EMERGENCY FLOAT SYSTEM |  |  |
| - CARGO HOOK SYSTEM |  |  |
| - SETTLING PROTECTORS |  |  |
| - Inspect settling protectors for condition. | 804--3 |  |
| - FIXED LANDING LIGHT -- FUSELAGE |  |  |
| - FIXED LANDING LIGHT -- CROSS TUBE |  |  |
| - . |  |  |
| - STROBE LIGHT SYSTEM |  |  |
| - Inspect strobe light system for general condition. | 810--5 step 1. And step 2. |  |
| - BOARDING STEP -- LONG |  |  |
| - . |  |  |
| - MAIN ROTOR BLADE FOLDING SYSTEM |  |  |
| - Remove, clean and inspect safety lever assemblies for condition (crack inspection not required). | 812--3 |  |
| - EXTERNAL LOUDSPEAKER SYSTEM |  |  |
| - . |  |  |
| - AFT BATTERY |  |  |
| - . |  |  |
| - RADOM |  |  |
| - . |  |  |
| - PROTECTIVE COVERS FOR DUAL CONTROLS |  |  |
| - . |  |  |
| - SNOW SKIDS |  |  |
| - . |  |  |
| - SEARCHLIGHT |  |  |
| - . |  |  |
| - EXTERIOR MIRROR |  |  |
| - . |  |  |
| - CARTRIDGE--TYPE FUEL PUMP |  |  |
| - Check fuel pump for condition. | 820--3 |  |
| - P/R CSAS SYSTEM |  |  |
| - . |  |  |
| - COPILOT‘S FLIGHT INSTRUMENTS |  |  |
| - . |  |  |
| - DUAL AC SYSTEM |  |  |
| - . |  |  |
| - INSTANTENOUS VERTICAL SPEED INDICATOR |  |  |
| - . |  |  |
| - EMERGENCY POWER SUPPLY -- STANDBY HORIZON |  |  |
| - . |  |  |
| - HF--ANTENNA |  |  |
| - . |  |  |
| - LANDING AND SEARCHLIGHT SYSTEM 450W |  |  |
| - . |  |  |
| - ENGINE CLEANING PROVISION |  |  |
| - Inspect engine water washing equipment for condition. | 830--2 |  |
| - SAND FILTER |  |  |
| - . |  |  |
| - DIGITAL AUTOMATIC FLIGHT CONTROL SYSTEM (DAFCS) |  |  |
| - . |  |  |
| - EMERGENCY FLOAT SYSTEM, MAX TOW 3350 KG |  |  |
| - . |  |  |
| - SCAVENGE OIL FILTER SYSTEM |  |  |
| - . |  |  |
| - HIGH LANDING GEAR |  |  |
| - . |  |  |
| - WIRE STRIKE PROTECTION SYSTEM (WSPS) |  |  |
| - . |  |  |
| - EMERGENCY POWER SUPPLY -- STANDBY HORIZON |  |  |
| Note; |  |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - POST INSPECTION |  |  |
| - Ensure applicable servicing requirements have been carried out. |  |  |
| - Check oil level of transmission, hydraulic tank, engine and rail rotor gearbox prior to running helicopter. |  |  |
| - Complete a ground run at 100% NR to check for leaks and confirm system operation and parameters are within Flight Manual limitations. – Qualified pilot only. |  |  |
| - Replace or close all applicable trim panels, covers, access and inspection doors/covers as necessary. |  |  |
| - Perform Functional Check Flight if required per BK-117 Maintenance Manual or RFM as appropriate. |  |  |
| - Make an entry in the appropriate logbook (refer to the following page for wording of approval for return to service). |  |  |

Upon satisfactory completion of each inspection item, the mechanic shall initial the pertinent space as provided.  All mechanics shall sign and initial, in the space provided, at the end of the form.

At the completion of each inspection form, an A&P mechanic shall check the entire form for completion and then enter his/her signature, certificate type and number, and date in the space provided at the end of the form.  The same individual is also responsible for making the logbook entry shown at the conclusion of this inspection.

I certify that I have inspected and/or completed all the items initialed by me in accordance with Maritime Helicopters’ Airbus MBB BK-117 Series Approved Aircraft Inspection Program and found them to be in an airworthy condition except as may be noted in the Aircraft Logbook.

| INITIALS | SIGNATURE | CERTIFICATE TYPE & NUMBER |
| --- | --- | --- |

I have reviewed this inspection form to verify that all items have been completed and have made an entry in the Aircraft Log Book.

I certify that this aircraft has been inspected in accordance with Maritime Helicopters’ Airbus MBB BK-117 Series AAIP – A 0030 – 50 Hour / 24 Month Inspection and was determined to be in an airworthy condition.

| Registration No.: | N | End |  | Aircraft Ending Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Log Page Number: |  |

| Signature: |  |  | Certificate Type & No: |  |
| --- | --- | --- | --- | --- |