| Registration No.: | N | Start |  | Aircraft Start Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Location: |  |

Reference:  Eurocopter B0-117 Series MM Appendix A

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - GENERAL |  |  |
| - Logbook review for recorded discrepancies. Correct discrepancies on installed equipment. |  |  |
| - Ensure all applicable and required Special Inspections, Alert Service Bulletins (ASB’s), Engine Service Bulletins, and Airworthiness Directives (AD’s) are accomplished. |  |  |
| - Replace all life limited components that have completed published operating limits. | Airbus |  |
|  | BK-117 MP |  |
| - Overhaul all components that have completed published operating limits. | Airbus |  |
|  | BK-117 MP |  |
| - PLACARDS AND MARKINGS |  |  |
| - Examine the placards, decals, and markings. Make sure you can read them, they are applied correctly, and they are in agreement with the applicable configuration of your helicopter. |  |  |
| - CORROSION CONTROL |  |  |
| - Perform corrosion inspection in any area’s access during this AAIP inspection. |  |  |
| - MAIN TRANSMISSION |  |  |
| - Use magnifying glass to inspect underside of rotor mast flange in the area of bores as well as webs between bores for cracks. |  |  |
| Effectivity; If washers P/N 105--13101.16 are installed |  |  |
| - ROTOR BRAKE SYSTEM |  |  |
| - Inspect brake pads for movability by hand (Rotor brake not applied) | 12-3, step 2 |  |
| - Change oil. | 12-16 thru 12-18 |  |
| - MAIN ROTOR SYSTEM |  |  |
| - MAIN ROTOR BLADE |  |  |
| - FUSELAGE |  |  |
| - COWLINGS |  |  |
| - Visually inspect goosenecks P/N 117-23236-01, P/N 117-23237-01of transmission cowling for cracks | 22-9 thru 22-11 |  |
| - TAIL BOOM |  |  |
| - Visually inspect tail rotor transmission mounting brackets in the vertical fin for loose rivets and cracks and adjacent areas for cracks. | 31-2, step 6 |  |
| - TAIL ROTOR DRIVE |  |  |
| - Remove magnetic plug of intermediate gear box and tail rotor transmission and inspect for deposits. | 32-32, step 3 |  |
| Effectivity; All BK117 if SB MBB--BK117--90--119 has not been implemented |  |  |
| - Inspect tail rotor transmission hardware for proper torque and tighten as necessary. | 32-37 |  |
| Effectivity; MBB--BK117 up to and including S/N 7261, if ASB MBB--BK117--30--104 has not been implemented |  |  |
| - TAIL ROTOR |  |  |
| - Without disassembly, inspect spherical bearings in fork end of tail rotor shaft for play, corrosion and binding. | 33-21, step 1 |  |
| - Inspect flapping hinge function. | 33-4, step 3 |  |
| - Dynamically balance the tail rotor. | 33-3 |  |
| - Inspect the bearings of the pitch links. | 33-7 step 3 |  |
| - TAIL ROTOR BLADE |  |  |
| - Remove the tail rotoblades. Inspect erosion protection for debonds. Inspect attachment area for damages. Inspect tail rotor blades for cracks in surface protection. It is not necessary to remove the cover tape in the nose shell area. | 34-2 step 2 and 7 |  |
| - Inspect tail rotor blade for impact damage and cracks in surface protection. The cover tape in the nose area does not have to removed. | 34-2, steps 2. |  |
| Effectivity; All tail rotor blades except P/N: 117-31743, 117-317411, B641M1001 101 |  |  |

| - MAIN ROTOR FLIGHT CONTROLS |  |  |
| --- | --- | --- |
| - Inspect attachment bolts between rotating control rods and control levers of main rotor head for security of mounting. | 41-3 step 4e |  |
| Effectivity; If lock washers P/N 117--142041.04 or P/N 105--13101.25 are not installed. |  |  |
| - TAIL ROTOR FLIGHT CONTROLS |  |  |
| - Inspect yaw actuator |  |  |
| Effectivity; Actuator P/N 741B 0000--05 and P/N 741C 0000--05 only |  |  |
| - HYDRAULIC SYSTEM |  |  |
| - LANDING GEAR |  |  |
| - FUEL SYSTEM |  |  |
| - LUBRICATION SYSTEM |  |  |
| - FIRE WALLS |  |  |
| - ENGINE OPERATION AND CONTROL |  |  |
| - STANDARD EQUIPMENT |  |  |
| - INSTRUMENT SYSTEMS |  |  |
| - ELECTRICAL SYSTEMS |  |  |
| - DC SUPPLY SYSTEM |  |  |
| - EMERGENCY FLOAT SYSTEM |  |  |
| - CARGO HOOK SYSTEM |  |  |
| - SETTLING PROTECTORS |  |  |
| - FIXED LANDING LIGHT -- FUSELAGE |  |  |
| - FIXED LANDING LIGHT -- CROSS TUBE |  |  |
| - . |  |  |
| - STROBE LIGHT SYSTEM |  |  |
| - . |  |  |
| - BOARDING STEP -- LONG |  |  |
| - . |  |  |
| - MAIN ROTOR BLADE FOLDING SYSTEM |  |  |
| - . |  |  |
| - EXTERNAL LOUDSPEAKER SYSTEM |  |  |
| - . |  |  |
| - AFT BATTERY |  |  |
| - . |  |  |
| - RADOM |  |  |
| - . |  |  |
| - PROTECTIVE COVERS FOR DUAL CONTROLS |  |  |
| - . |  |  |
| - SNOW SKIDS |  |  |
| - . |  |  |
| - SEARCHLIGHT |  |  |
| - . |  |  |
| - EXTERIOR MIRROR |  |  |
| - . |  |  |
| - CARTRIDGE--TYPE FUEL PUMP |  |  |
| - . |  |  |
| - P/R CSAS SYSTEM |  |  |
| - . |  |  |
| - COPILOT‘S FLIGHT INSTRUMENTS |  |  |
| - . |  |  |
| - DUAL AC SYSTEM |  |  |
| - . |  |  |
| - INSTANTENOUS VERTICAL SPEED INDICATOR |  |  |
| - . |  |  |
| - EMERGENCY POWER SUPPLY -- STANDBY HORIZON |  |  |
| - . |  |  |
| - HF--ANTENNA |  |  |
| - . |  |  |
| - LANDING AND SEARCHLIGHT SYSTEM 450W |  |  |
| - . |  |  |
| - ENGINE CLEANING PROVISION |  |  |
| - . |  |  |
| - SAND FILTER |  |  |
| - . |  |  |
| - DIGITAL AUTOMATIC FLIGHT CONTROL SYSTEM (DAFCS) |  |  |
| - . |  |  |
| - EMERGENCY FLOAT SYSTEM, MAX TOW 3350 KG |  |  |
| - . |  |  |
| - SCAVENGE OIL FILTER SYSTEM |  |  |
| - . |  |  |
| - HIGH LANDING GEAR |  |  |
| - . |  |  |
| - WIRE STRIKE PROTECTION SYSTEM (WSPS) |  |  |
| - . |  |  |
| - EMERGENCY POWER SUPPLY -- STANDBY HORIZON |  |  |
| Note; |  |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - POST INSPECTION |  |  |
| - Ensure applicable servicing requirements have been carried out. |  |  |
| - Check oil level of transmission, hydraulic tank, engine and rail rotor gearbox prior to running helicopter. |  |  |
| - Complete a ground run at 100% NR to check for leaks and confirm system operation and parameters are within Flight Manual limitations. – Qualified pilot only. |  |  |
| - Replace or close all applicable trim panels, covers, access and inspection doors/covers as necessary. |  |  |
| - Perform Functional Check Flight if required per BK-117 Maintenance Manual or RFM as appropriate. |  |  |
| - Make an entry in the appropriate logbook (refer to the following page for wording of approval for return to service). |  |  |

Upon satisfactory completion of each inspection item, the mechanic shall initial the pertinent space as provided.  All mechanics shall sign and initial, in the space provided, at the end of the form.

At the completion of each inspection form, an A&P mechanic shall check the entire form for completion and then enter his/her signature, certificate type and number, and date in the space provided at the end of the form.  The same individual is also responsible for making the logbook entry shown at the conclusion of this inspection.

I certify that I have inspected and/or completed all the items initialed by me in accordance with Maritime Helicopters’ Airbus MBB BK-117 Series Approved Aircraft Inspection Program and found them to be in an airworthy condition except as may be noted in the Aircraft Logbook.

| INITIALS | SIGNATURE | CERTIFICATE TYPE & NUMBER |
| --- | --- | --- |

I have reviewed this inspection form to verify that all items have been completed and have made an entry in the Aircraft Log Book.

I certify that this aircraft has been inspected in accordance with Maritime Helicopters’ Airbus MBB BK-117 Series AAIP – A 0040 – 50 Hour / 24 Month Inspection and was determined to be in an airworthy condition.

| Registration No.: | N | End |  | Aircraft Ending Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Log Page Number: |  |

| Signature: |  |  | Certificate Type & No: |  |
| --- | --- | --- | --- | --- |