| Registration No.: | N | Start |  | Aircraft Start Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Location: |  |

Reference:  Eurocopter B0-117 Series MM Appendix A

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - GENERAL |  |  |
| - Logbook review for recorded discrepancies. Correct discrepancies on installed equipment. |  |  |
| - Ensure all applicable and required Special Inspections, Alert Service Bulletins (ASB’s), Engine Service Bulletins, and Airworthiness Directives (AD’s) are accomplished. |  |  |
| - Replace all life limited components that have completed published operating limits. | Airbus |  |
|  | BK-117 MP |  |
| - Overhaul all components that have completed published operating limits. | Airbus |  |
|  | BK-117 MP |  |
| - PLACARDS AND MARKINGS |  |  |
| - Examine the placards, decals, and markings. Make sure you can read them, they are applied correctly, and they are in agreement with the applicable configuration of your helicopter. |  |  |
| - CORROSION CONTROL |  |  |
| - Perform corrosion inspection in any area’s access during this AAIP inspection. |  |  |
| - MAIN TRANSMISSION |  |  |
| - Change oil, inspect and clean oil filters. | 11-4, 11-10 thru 11-13 |  |
| - Remove magnetic plug and inspect for deposits, clean and test for magnetic retaining force - Requires special weighted ball bearings. | 11-14 thru 11-17 02-14 |  |
| - Slider guide - Inspect transition radius in the flange area for cracks |  |  |
| Effectivity: P/N 4639305053, P/N 4639305030 |  |  |
| - ROTOR BRAKE SYSTEM |  |  |
| - MAIN ROTOR SYSTEM |  |  |
| - MAIN ROTOR BLADE |  |  |
| - FUSELAGE |  |  |
|  | 21-5, -7, -9(A), -11(A) |  |
| - Check jettison device of cockpit doors and cabin doors for function. |  |  |
| - Inspect the cabin doors for condition | 21--3 step 21A |  |
| - Inspect tail boom attachment for condition (frame 1L and 2L). |  |  |
| - Remove tail boom and inspect frame (STA 6616) for condition |  |  |
| EFFECTIVITY: MBB--BK117 up to and including S/N 7250, if SB MBB--BK117--20--101 has not been implemented: |  |  |
| - Ensure drain holes are not obstructed. |  |  |
| - COWLINGS |  |  |
| - TAIL BOOM |  |  |
| - TAIL ROTOR DRIVE |  |  |
| - TAIL ROTOR |  |  |
| - Dynamically balance the tail rotor. | 33-3 |  |
| - Inspect the bearings of the pitch links. | 33-7 step 3 |  |
| - TAIL ROTOR BLADE |  |  |
| - Remove the tail rotoblades. Inspect erosion protection for debonds. Inspect attachment area for damages. Inspect tail rotor blades for cracks in surface protection. It is not necessary to remove the cover tape in the nose shell area. | 34-2 step 2 and 7 |  |
| - MAIN ROTOR FLIGHT CONTROLS |  |  |
| - TAIL ROTOR FLIGHT CONTROLS |  |  |
| - HYDRAULIC SYSTEM |  |  |
| - LANDING GEAR |  |  |
| - FUEL SYSTEM |  |  |
| - LUBRICATION SYSTEM |  |  |
| - FIRE WALLS |  |  |
| - ENGINE OPERATION AND CONTROL |  |  |
| - STANDARD EQUIPMENT |  |  |
| - INSTRUMENT SYSTEMS |  |  |
| - ELECTRICAL SYSTEMS |  |  |
| - Inspect ground leads and ground connections of fueling nozzle and fuselage/landing gear for condition. |  |  |
| - Inspect bonding connections and bonding points for condition without disassembling. | 02--15 |  |
| - Inspect bonding connections and bonding wire for condition without disassembling. | 92--33 step 2 and 02--15 |  |
| - DUAL AC SYSTEM |  |  |
| - Inspect system for condition. | 823--5 |  |
| - Test conducting and blocking function of diodes. | 823--8 |  |
| EFFECTIVITY: Only applicable to helicopters S/N 7183 and prior, equipped with DAFCS system. |  |  |
| - EMERGENCY FLOAT SYSTEM |  |  |
| - CARGO HOOK SYSTEM |  |  |
| - SETTLING PROTECTORS |  |  |
| - FIXED LANDING LIGHT -- FUSELAGE |  |  |
| - FIXED LANDING LIGHT -- CROSS TUBE |  |  |
| - . |  |  |
| - STROBE LIGHT SYSTEM |  |  |
| - . |  |  |
| - BOARDING STEP -- LONG |  |  |
| - . |  |  |
| - MAIN ROTOR BLADE FOLDING SYSTEM |  |  |
| - . |  |  |
| - EXTERNAL LOUDSPEAKER SYSTEM |  |  |
| - . |  |  |
| - AFT BATTERY |  |  |
| - . |  |  |
| - RADOM |  |  |
| - . |  |  |
| - PROTECTIVE COVERS FOR DUAL CONTROLS |  |  |
| - . |  |  |
| - SNOW SKIDS |  |  |
| - . |  |  |
| - SEARCHLIGHT |  |  |
| - . |  |  |
| - EXTERIOR MIRROR |  |  |
| - . |  |  |
| - CARTRIDGE--TYPE FUEL PUMP |  |  |
| - . |  |  |
| - P/R CSAS SYSTEM |  |  |
| - Inspect P/R CSAS for condition. | 821--4 |  |
| - COPILOT‘S FLIGHT INSTRUMENTS |  |  |
| - Inspect instrument lighting -- copilot for condition. | 822--5 |  |
| - DUAL AC SYSTEM |  |  |
| - . |  |  |
| - INSTANTENOUS VERTICAL SPEED INDICATOR |  |  |
| - . |  |  |
| - EMERGENCY POWER SUPPLY -- STANDBY HORIZON |  |  |
| - . |  |  |
| - HF--ANTENNA |  |  |
| - . |  |  |
| - LANDING AND SEARCHLIGHT SYSTEM 450W |  |  |
| - . |  |  |
| - ENGINE CLEANING PROVISION |  |  |
| - . |  |  |
| - SAND FILTER |  |  |
| - . |  |  |
| - DIGITAL AUTOMATIC FLIGHT CONTROL SYSTEM (DAFCS) |  |  |
| - . |  |  |
| - EMERGENCY FLOAT SYSTEM, MAX TOW 3350 KG |  |  |
| - . |  |  |
| - SCAVENGE OIL FILTER SYSTEM |  |  |
| - . Remove chip detector and test magnetic retaining force. | 836--9, 02--14, 836--10 |  |
| - HIGH LANDING GEAR |  |  |
| - . |  |  |
| - WIRE STRIKE PROTECTION SYSTEM (WSPS) |  |  |
| - . |  |  |
| - EMERGENCY POWER SUPPLY -- STANDBY HORIZON |  |  |
| Note; |  |  |
|  |  | MECHANIC’S INITIALS |
| MAINTENANCE ACTION | REFERENCE |  |
| - POST INSPECTION |  |  |
| - Ensure applicable servicing requirements have been carried out. |  |  |
| - Check oil level of transmission, hydraulic tank, engine and rail rotor gearbox prior to running helicopter. |  |  |
| - Complete a ground run at 100% NR to check for leaks and confirm system operation and parameters are within Flight Manual limitations. – Qualified pilot only. |  |  |
| - Replace or close all applicable trim panels, covers, access and inspection doors/covers as necessary. |  |  |
| - Perform Functional Check Flight if required per BK-117 Maintenance Manual or RFM as appropriate. |  |  |
| - Make an entry in the appropriate logbook (refer to the following page for wording of approval for return to service). |  |  |

Upon satisfactory completion of each inspection item, the mechanic shall initial the pertinent space as provided.  All mechanics shall sign and initial, in the space provided, at the end of the form.

At the completion of each inspection form, an A&P mechanic shall check the entire form for completion and then enter his/her signature, certificate type and number, and date in the space provided at the end of the form.  The same individual is also responsible for making the logbook entry shown at the conclusion of this inspection.

I certify that I have inspected and/or completed all the items initialed by me in accordance with Maritime Helicopters’ Airbus MBB BK-117 Series Approved Aircraft Inspection Program and found them to be in an airworthy condition except as may be noted in the Aircraft Logbook.

| INITIALS | SIGNATURE | CERTIFICATE TYPE & NUMBER |
| --- | --- | --- |

I have reviewed this inspection form to verify that all items have been completed and have made an entry in the Aircraft Log Book.

I certify that this aircraft has been inspected in accordance with Maritime Helicopters’ Airbus MBB BK-117 Series AAIP – A 0090 – 50 Hour / 24 Month Inspection and was determined to be in an airworthy condition.

| Registration No.: | N | End |  | Aircraft Ending Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Log Page Number: |  |

| Signature: |  |  | Certificate Type & No: |  |
| --- | --- | --- | --- | --- |