| Registration No.: | N | Start |  | Aircraft Start Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Location: |  |

Reference:  Eurocopter B0-117 Series MM Appendix A

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - GENERAL |  |  |
| - Logbook review for recorded discrepancies. Correct discrepancies on installed equipment. |  |  |
| - Ensure all applicable and required Special Inspections, Alert Service Bulletins (ASB’s), Engine Service Bulletins, and Airworthiness Directives (AD’s) are accomplished. |  |  |
| - Replace all life limited components that have completed published operating limits. | Airbus |  |
|  | BK-117 MP |  |
| - Overhaul all components that have completed published operating limits. | Airbus |  |
|  | BK-117 MP |  |
| - PLACARDS AND MARKINGS |  |  |
| - Examine the placards, decals, and markings. Make sure you can read them, they are applied correctly, and they are in agreement with the applicable configuration of your helicopter. |  |  |
| - CORROSION CONTROL |  |  |
| - Perform corrosion inspection in any area’s access during this AAIP inspection. |  |  |
| - MAIN TRANSMISSION |  |  |
| - ROTOR BRAKE SYSTEM |  |  |
| - Change oil of rotor brake. | 12--4 |  |
| - MAIN ROTOR SYSTEM |  |  |

| - MAIN ROTOR BLADE |  |  |
| --- | --- | --- |
| - Perform lead--lag check or alternatively inspect the blade thimble area. |  |  |
| Effectivity: Main rotor blades (blade sweep angle 1 degree) P/N 117--150001, P/N 117--150011 and all versions, 117--150021 and all versions, 117--50061 and all versions, 117--151321 and all versions, 117--151341 and all versions, 117--151351 and all versions, 117--151361 and all versions. |  |  |
| - Inspect main rotor blades for bulges in the vicinity of the lead balance weights. | 14--2, step 5 |  |
| Effectivity: Blade fittings P/N 117--15854.05, 117--15860.05, 117--15854.06 and 117--15860.06 |  |  |
| - FUSELAGE |  |  |
| - COWLINGS |  |  |
| - TAIL BOOM |  |  |
| - TAIL ROTOR DRIVE |  |  |
| - TAIL ROTOR |  |  |
| - TAIL ROTOR BLADE |  |  |
| - MAIN ROTOR FLIGHT CONTROLS |  |  |
| - Inspect bearing forks of swashplate control ring and swashplate bearing ring for cracks | 41-3 |  |
| - TAIL ROTOR FLIGHT CONTROLS |  |  |
| - HYDRAULIC SYSTEM |  |  |
| - Inspect rod end seals for damage. In case of damaged seals, remove control rods, disassemble and inspect for condition. | 43-32 thru 43-37 |  |
| - Bleed both reservoirs. | 43-5 |  |
| - Inspect hydraulic system for condition. | 43-11 |  |
| - Change hydraulic fluid. | 43-6 |  |
| - Replace filter elements. | 43-28, 43-29 |  |
| - Do the leakage check of the hydraulic system. | 43-12 |  |
| - Do the functional test of the hydraulic system. | 43-14 |  |
| - Grease splines of hydraulic pump drive shaft. | 43-20, 43-22 |  |
| - FUEL SYSTEM |  |  |
| - LUBRICATION SYSTEM |  |  |
| - FIRE WALLS |  |  |
| - ENGINE OPERATION AND CONTROL |  |  |
| - STANDARD EQUIPMENT |  |  |
| - INSTRUMENT SYSTEMS |  |  |
| - ELECTRICAL SYSTEMS |  |  |
| - DC SUPPLY SYSTEM |  |  |
| - EMERGENCY FLOAT SYSTEM |  |  |
| - CARGO HOOK SYSTEM |  |  |
| - SETTLING PROTECTORS |  |  |
| - FIXED LANDING LIGHT -- FUSELAGE |  |  |
| - FIXED LANDING LIGHT -- CROSS TUBE |  |  |
| - . |  |  |
| - STROBE LIGHT SYSTEM |  |  |
| - . |  |  |
| - MAIN ROTOR BLADE FOLDING SYSTEM |  |  |
| - . |  |  |
| - EXTERNAL LOUDSPEAKER SYSTEM |  |  |
| - AFT BATTERY |  |  |
| - . |  |  |
| - RADOM |  |  |
| - . |  |  |
| - PROTECTIVE COVERS FOR DUAL CONTROLS |  |  |
| - . |  |  |
| - SEARCHLIGHT |  |  |
| - . |  |  |
| - EXTERIOR MIRROR |  |  |
| - . |  |  |
| - CARTRIDGE--TYPE FUEL PUMP |  |  |
| - . |  |  |
| - P/R CSAS SYSTEM |  |  |
| - . |  |  |
| - COPILOT‘S FLIGHT INSTRUMENTS |  |  |
| - . |  |  |
| - DUAL AC SYSTEM |  |  |
| - . |  |  |
| - INSTANTENOUS VERTICAL SPEED INDICATOR |  |  |
| - . |  |  |
| - EMERGENCY POWER SUPPLY -- STANDBY HORIZON |  |  |
| - . |  |  |
| - HF--ANTENNA |  |  |
| - . |  |  |
| - ENGINE CLEANING PROVISION |  |  |
| - . |  |  |
| - SAND FILTER |  |  |
| - . |  |  |
| - DIGITAL AUTOMATIC FLIGHT CONTROL SYSTEM (DAFCS) |  |  |
| - . |  |  |
| - EMERGENCY FLOAT SYSTEM, MAX TOW 3350 KG |  |  |
| - . |  |  |
| - SCAVENGE OIL FILTER SYSTEM |  |  |
| - . |  |  |
| - WIRE STRIKE PROTECTION SYSTEM (WSPS) |  |  |
| - . |  |  |
| - EMERGENCY POWER SUPPLY -- STANDBY HORIZON |  |  |
| Note; |  |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - POST INSPECTION |  |  |
| - Ensure applicable servicing requirements have been carried out. |  |  |
| - Check oil level of transmission, hydraulic tank, engine and rail rotor gearbox prior to running helicopter. |  |  |
| - Complete a ground run at 100% NR to check for leaks and confirm system operation and parameters are within Flight Manual limitations. – Qualified pilot only. |  |  |
| - Replace or close all applicable trim panels, covers, access and inspection doors/covers as necessary. |  |  |
| - Perform Functional Check Flight if required per BK-117 Maintenance Manual or RFM as appropriate. |  |  |
| - Make an entry in the appropriate logbook (refer to the following page for wording of approval for return to service). |  |  |

Upon satisfactory completion of each inspection item, the mechanic shall initial the pertinent space as provided.  All mechanics shall sign and initial, in the space provided, at the end of the form.

At the completion of each inspection form, an A&P mechanic shall check the entire form for completion and then enter his/her signature, certificate type and number, and date in the space provided at the end of the form.  The same individual is also responsible for making the logbook entry shown at the conclusion of this inspection.

I certify that I have inspected and/or completed all the items initialed by me in accordance with Maritime Helicopters’ Airbus MBB BK-117 Series Approved Aircraft Inspection Program and found them to be in an airworthy condition except as may be noted in the Aircraft Logbook.

| INITIALS | SIGNATURE | CERTIFICATE TYPE & NUMBER |
| --- | --- | --- |

I have reviewed this inspection form to verify that all items have been completed and have made an entry in the Aircraft Log Book.

I certify that this aircraft has been inspected in accordance with Maritime Helicopters’ Airbus MBB BK-117 Series AAIP – A 0110 – 50 Hour / 24 Month Inspection and was determined to be in an airworthy condition.

| Registration No.: | N | End |  | Aircraft Ending Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Log Page Number: |  |

| Signature: |  |  | Certificate Type & No: |  |
| --- | --- | --- | --- | --- |