| Registration No.: | N | Start |  | Aircraft Start Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Location: |  |

Reference:  Eurocopter B0-117 Series MM Appendix A

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - GENERAL |  |  |
| - Logbook review for recorded discrepancies. Correct discrepancies on installed equipment. |  |  |
| - Ensure all applicable and required Special Inspections, Alert Service Bulletins (ASB’s), Engine Service Bulletins, and Airworthiness Directives (AD’s) are accomplished. |  |  |
| - Replace all life limited components that have completed published operating limits. | Eurocopter |  |
|  | BO-117 MP |  |
| - Overhaul all components that have completed published operating limits. | Eurocopter |  |
|  | BO-117 MP |  |
| - PLACARDS AND MARKINGS |  |  |
| - Examine the placards, decals, and markings. Make sure you can read them, they are applied correctly, and they are in agreement with the applicable configuration of your helicopter. |  |  |
| - CORROSION CONTROL |  |  |
| - Perform corrosion inspection in any area’s access during this AAIP inspection. |  |  |
| - MAIN TRASMISSION |  |  |
|  | 11-3 steps 1, 2 and 4 thru 6 |  |
| - Inspect main transmission for condition. |  | A0010 |
| - Remove magnetic plug and inspect for deposits, clean and test for magnetic retaining force - Requires special weighted ball bearings. | 11-14 thru 11-17 02-14 | A0090 |
| - Slider guide - Inspect transition radius in the flange area for cracks |  | A0090 |
| Effectivity: P/N 4639305053, P/N 4639305030 |  |  |
| - ROTOR BRAKE SYSTEM |  |  |
| - Inspect rotor brake system for condition | 12-3 | A0020 |
| - MAIN ROTOR SYSTEM |  |  |
| - Inspect main rotor head for condition; for this remove rotor hub cap. | 13-5 | A0120 |
| - Remove primary bolts, clean and inspect for condition (crack inspection not necessary) | 13-10, 13-11 | A0120 |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - Remove secondary bolts, clean and inspect for condition (crack inspection not necessary) | 13-17, 13-18 | A0120 |
| - MAIN ROTOR BLADE |  |  |
| - Inspect main rotor blades for condition | 14-2 | A0120 |
| Inspect blade thimble area. | 14--9 thru 14--11 | A0120 |
| NOTE: When accomplishing the inspection of the blade thimble area it is not necessary to perform the lead--lag check. |  |  |
| - FUSELAGE |  |  |
|  | 21-5, -7, -9(A), -11(A) |  |
| - Check jettison device of cockpit doors and cabin doors for function. |  | A0090 |
| - Inspect the cabin doors for condition | 21--3 step 21A | A0090 |
| - Inspect tail boom attachment for condition (frame 1L and 2L). |  | A0090 |
| - Remove tail boom and inspect frame (STA 6616) for condition |  | A0090 |
| EFFECTIVITY: MBB--BK117 up to and including S/N 7250, if SB MBB--BK117--20--101 has not been implemented: |  |  |
| - Ensure drain holes are not obstructed. |  |  |
| - COWLINGS |  |  |
| - Visually inspect goosenecks for cracks. | 22-9 thru 22-11 | A0020 |
| - Check latch of engine cowlings and transmission cowlings for function | 22--10, 22--14 | A0010 |
| EFFECTIVITY: After performance of SB--MBB--BK 117--20--109 |  |  |
| - TAILBOOM |  |  |
| - Inspect tail boom assembly for condition. | 31-2 | A0100 |
| - TAIL ROTOR DRIVE |  |  |
|  | 32-37 |  |
| - Inspect tail rotor gear box attaching hardware for cracks of the self--locking nuts and for proper torque and tighten as necessary |  | A0060 |
| - Inspect tail rotor drive for condition. | 32-3 | A0060 |
| - Test magnetic plugs of intermediate gearbox and tail rotor gearbox for magnetic retaining force - Requires special weighted ball bearings. | 32-32 step 3, 02-14 | A0060 |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - TAIL ROTOR |  |  |
|  | 33-4 steps 1 thru 7 and 9 thru 13 |  |
| - Inspect tail rotor for condition. |  | A0070 |
| - Remove bellcrank and inspect for condition. | 33-10 thru 33-12 | A0070 |
| - Remove bolts P/N 105-31717 of bellcrank and inspect for condition. | 33-9 | A0070 |
| - Remove bolts attached to tail rotor shaft/ tail rotor head and inspect for condition. | 33-21 step 1.e | A0070 |
| - Without disassembly, inspect spherical bearings in fork end of tail rotor shaft for play, corrosion and binding | 33-21 step 1.c | A0020, A0070 |
| - TAIL ROTOR BLADE |  |  |
| - Inspect tail rotor blades for condition. | 34-2 | A0070 |
| - MAIN ROTOR FLIGHT CONTROLS |  |  |
| - Inspect flight control components (boosted section of flight controls) located above hydraulic system including swash plate for condition. | 41-3 Step 4 | A0050 |
| - Clean sliding surface of swash plate on sliding guide. |  | A0050 |
| - Inspect swash plate bearing for rough running. | 41-52 | A0050 |
| - Inspect upper rod end seals for damage. | 41-59 step 6 | A0050 |
| - Check friction forces of pitch brake. | 41-14 step 5 | A0050 |
| - TAIL ROTOR CONTROLS |  |  |
| - Inspect upper rod end seals for damage. | 42-31 step 4 | A0120 |
| - HYDRAULIC SYSTEM |  |  |
| - Inspect rod end seals for damage. In case of damaged seals, remove control rods, disassemble and inspect for condition. | 43-32 thru 43-37 | A0110 |
| - Bleed both reservoirs. | 43-5 | A0110 |
| - Inspect hydraulic system for condition. | 43-11 | A0110 |
| - Change hydraulic fluid. | 43-6 | A0110 |
| - Do the leakage check of the hydraulic system. | 43-12 | A0110 |
| - Do the functional test of the hydraulic system. | 43-14 | A0110 |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - LANDING GEAR |  |  |
| - Inspect landing gear for condition. | 51-2 | A0110 |
| - Inspect the cross tubes for corrosion. | 51-2 | A0110 |
| - FUEL SYSTEM |  |  |
| - Check fuel shutoff valves for function and evidence of leakage. | 62-39 step 5 | A0030 |
| - Inspect fuel system for leakage. | 62-40 | A0030 |
| - LUBRICATION SYSTEM |  |  |
| - Remove chip detectors, clean and test magnetic retaining force - Requires special weighted ball bearings. | 63-21, 02-14, 63-23 |  |
| - Inspect lubricating system for condition. | 63-3 |  |
| - FIRE WALLS |  |  |
| - Inspect fire walls for condition. | 64-2 | A0070 |
| - Inspect ejector for condition. | 64-10 | A0070 |
| - ENGINE OPERATION AND CONTROL |  |  |
| - Inspect engine operation and controls for condition. | 65-3 |  |
| - STANDARD EQUIPMENT |  |  |
| - Inspect identification and lettering for readability, placards for damage and security of attachment. | IPC -- FLM | A0060 |
| - Inspect windshield wiper system for condition and function. | 702-3, 702-4 | A0060 |
| - Inspect fire extinguishing system for condition and function. | 706-3, 706-4 | A0060 |
| - Inspect YAW system for condition. | 709-3 | A0060 |
| - Inspect hand fire extinguisher for security of attachment and proper charge. |  | A0060 |
| - Inspect first aid kit for completeness of contents and security of attachment. |  | A0060 |
| - INSTRUMENT SYSTEMS |  |  |
| - Inspect instrument panel decals and all instrument markings for presence and readability. |  | A0070 |
| - Inspect visible parts of pitot static system for condition. | 91-23 | A0070 |
|  |  | MECHANIC’S INITIALS |
| MAINTENANCE ACTION | REFERENCE |  |
|  | 91-46, 91-47, 91-56 |  |
| - Compensate compasses and inspect for condition. |  | A0070 |
| - Inspect directional gyro for function. | 91-49 | A0070 |
| - Check the following; |  |  |
| - Pitot system for leakage. | 91-21 step 1 | A0070 |
| - Airspeed indicator for correct indication. | 91-21 step 2 | A0070 |
| - Static system for leakage | 91-21 step 3 | A0070 |
| - Altimeter for correct indication | 91-21 step 4 | A0070 |
| - ELECTRICAL SYSTEMS |  |  |
| - Inspect ground leads and ground connections of fueling nozzle and fuselage/landing gear for condition. |  | A0090 |
| - EMERGENCY FLOAT SYSTEM, MAX. TOW 2850 KG |  |  |
| - Check emergency float system for operational readiness. |  | A0050 |
| NOTE: Circuit breaker EMER FLOATS must be set and toggle switch EMER FLOATS must be in ARMED position. If green indicator light is not on, refer to para 802--2, TROUBLESHOOTING |  |  |
| - Inspect emergency float system for condition (floats removed). | 02--4 | A0050 |
| - Perform trial inflation. | Refer to manufacturer CMM 25--60--97 | A0050 |
| - SETTLING PROTECTORS |  |  |
| - Inspect settling protectors for condition. | 804--3 | A0030 |
| - FIXED LANDING LIGHT -- FUSELAGE |  |  |
| - Inspect fixed landing light for condition. | 08--5 | A0050 |
| - FIXED LANDING LIGHT -- CROSSTUBE |  |  |
| - Inspect landing light for general condition. | 08--4 | A0050 |
| - STROBE LIGHT SYSTEM |  |  |
|  | 810--5 step 1. And step 2. |  |
| - Inspect strobe light system for general condition |  | A0030 |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - BOARDING STEP -- LONG |  |  |
| - Inspect boarding step for condition. | 811--2 | A0110 |
| - MAIN ROTOR BLADE FOLDING SYSTEM |  |  |
| - Remove, clean and inspect safety lever assemblies for condition (crack inspection not required). | 812--3 | A0030 |
| - EXTERNAL LOUDSPEAKER SYSTEM |  |  |
| - Inspect external loudspeaker system for condition. | 813--4 | A0010 |
| - AFT BATTERY |  |  |
| - Check battery system for condition. | 814--3 | A0010 |
| - RADOM |  |  |
| - Remove Radom and inspect for condition. | 815--2 | A0010 |
| - PROTECTIVE COVERS FOR DUAL CONTROLS |  |  |
| - Check protective covers for condition. | 816--2 | A0010 |
| - SNOW SKIDS |  |  |
| - Check condition of snow skids. | 817--2 | A0110 |
| - SEARCHLIGHT |  |  |
| - Functional test of searchlight. | 818--4 | A0010 |
| - Inspect searchlight for condition | 818--5 | A0010 |
| - EXTERIOR MIRROR |  |  |
| - Check mirror for condition. | 819--3 | A0010 |
| - CARTRIDGE--TYPE FUEL PUMP |  |  |
| - Check fuel pump for condition. | 820--3 | A0030 |
| - P/R CSAS SYSTEM |  |  |
| - Inspect P/R CSAS for condition. | 821--4 | A0090 |
| - COPILOT’S FLIGHT INSTRUMENTS |  |  |
| - Inspect instrument lighting -- copilot for condition. | 822--5 | A0090 |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - DUAL AC SYSTEM |  |  |
| - Inspect system for condition. | 823--5 | A0090 |
| - Test conducting and blocking function of diodes. | 823--8 | A0090 |
| EFFECTIVITY: Only applicable to helicopters S/N 7183 and prior, equipped with DAFCS system. |  |  |
| - INSTANTANEOUS VERTICAL SPEED INDICATOR |  |  |
| - Inspect indicator for condition. | 825--4 | A0070 |
| - Examine indicators. |  | A0070 |
| - EMERGENCY POWER SUPPLY -- STANDBY HORIZON |  |  |
| - Perform discharge check. | 826--10 | A0070 |
| - HF--ANTENNA |  |  |
| - Inspect HF--antenna for condition. | 828--3 | A0050 |
| - Inspect antenna cable ends. | 828--4 | A0050 |
| - LANDING AND SEARCH LIGHT SYSTEM |  |  |
| - Check condition of system. | 829--4 | A0110 |
| - Carry out functional test on system. | 829--5 | A0110 |
| - ENGINE CLEANING PROVISION |  |  |
| - Inspect engine water washing equipment for condition. | 830--2 | A0030 |
| - SAND FILTER |  |  |
|  | 831--15 and 831--16 |  |
| - Remove and inspect bypass door actuators. |  | A0070 |
| - DIGITAL AUTOMATIC FLIGHT CONTROL SYSTEM (DAFCS) |  |  |
| - Inspect digital automatic flight control system for condition. | 833--4 | A0070 |
| - EMERGENCY FLOAT SYSTEM MAX. TOW 3350 KG |  |  |
|  | 835--4, without step 9 |  |
| - Inspect emergency float system for condition. |  | A0050 |
| - Perform trial inflation test. | Manufacturer’s Component Maintenance Manual CMM 25--63--04 |  |
|  |  | MECHANIC’S INITIALS |
| MAINTENANCE ACTION | REFERENCE |  |
| - SCAVENGE OIL FILTER SYSTEM |  |  |
|  | 836--9, 02--14, 836--10 |  |
| - Remove chip detector and test magnetic retaining force. |  | A0090 |
| - HIGH LANDING GEAR |  |  |
|  | 840--3 step 1. thru 14. and 16. |  |
| - Inspect high landing gear for condition. |  | A0110 |
| - WIRE STRIKE PROTECTION SYSTEM (WSPS) |  |  |
| - Inspect WSPS for condition. | 844--3 | A0010 |
| - POST INSPECTION |  |  |
| - Ensure applicable servicing requirements have been carried out. |  |  |
| - Check oil level of transmission, hydraulic tank, engine and rail rotor gearbox prior to running helicopter. |  |  |
| - Complete a ground run at 100% NR to check for leaks and confirm system operation and parameters are within Flight Manual limitations. – Qualified pilot only. |  |  |
| - Replace or close all applicable trim panels, covers, access and inspection doors/covers as necessary. |  |  |
| - Perform Functional Check Flight if required per BO 117 Maintenance Manual or RFM as appropriate. |  |  |
| - Make an entry in the appropriate logbook (refer to the following page for wording of approval for return to service). |  |  |

Upon satisfactory completion of each inspection item, the mechanic shall initial the pertinent space as provided.  All mechanics shall sign and initial, in the space provided, at the end of the form.

At the completion of each inspection form, an A&P mechanic shall check the entire form for completion and then enter his/her signature, certificate type and number, and date in the space provided at the end of the form.  The same individual is also responsible for making the logbook entry shown at the conclusion of this inspection.

I certify that I have inspected and/or completed all the items initialed by me in accordance with Maritime Helicopters’ Eurocopter BO-117 Series Approved Aircraft Inspection Program and found them to be in an airworthy condition except as may be noted in the Aircraft Logbook.

| INITIALS | SIGNATURE | CERTIFICATE TYPE & NUMBER |
| --- | --- | --- |

I have reviewed this inspection form to verify that all items have been completed and have made an entry in the Aircraft Log Book.

I certify that this aircraft has been inspected in accordance with Maritime Helicopters’ Eurocopter BO-117 Series AAIP – A 0624 – 600 Hour / 24 Month Inspection and was determined to be in an airworthy condition.

| Registration No.: | N | End |  | Aircraft Ending Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Log Page Number: |  |

| Signature: |  |  | Certificate Type & No: |  |
| --- | --- | --- | --- | --- |