| Registration No.: | N | Start |  | Aircraft Start Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Location: |  |

Reference:  Airbus BO-117 Series MM Appendix A

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - GENERAL |  |  |
| - Logbook review for recorded discrepancies. Correct discrepancies on installed equipment. |  |  |
| - Ensure all applicable and required Special Inspections, Alert Service Bulletins (ASB’s), Engine Service Bulletins, and Airworthiness Directives (AD’s) are accomplished. |  |  |
| - Replace all life limited components that have completed published operating limits. | Eurocopter |  |
|  | BO-117 MP |  |
| - Overhaul all components that have completed published operating limits. | Eurocopter |  |
|  | BO-117 MP |  |
| - PLACARDS AND MARKINGS |  |  |
| - Inspect identification and lettering for readability, placards for damage and security of attachment | IPC -- FLM |  |
| - CORROSION CONTROL |  |  |
| - Perform corrosion inspection in any area’s access during this AAIP inspection. |  |  |
| - MAIN TRASMISSION |  |  |
| - Remove magnetic plug and inspect for deposits, clean and test for magnetic retaining force - Requires special weighted ball bearings | 11-14 thru 11-17, 02-14 |  |
| - MAIN ROTOR BLADE |  |  |
| - Inspect transmission mounting struts for condition. | 14-2, step 11 h |  |
| EFFECTIVITY: Lower fitting P/N 117--80166.02: |  |  |
| - FUSELAGE |  |  |
| - Check jettison device for cockpit doors and cabin doors for function. | 21-5, -7, -9(A), |  |
|  | -11(A) |  |
| - Inspect the cabin doors for condition. | 21--3 step 2 |  |
| - Inspect the door spar for the cabin doors for condition. | 21--3 step 2.a |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - TAIL ROTOR DRIVE |  |  |
| - Remove magnetic plugs of intermediate gearbox and tail rotor gearbox and test their magnetic retaining force - Requires special weighted ball bearings | 32-32 step 3, 02-14 |  |
| - TAIL ROTOR |  |  |
| - Lubricate the following; |  |  |
| - Ball bearings of sliding sleeve. | 33-27 step 13 |  |
| - Splines of tail rotor shaft. |  |  |
| - MAIN ROTOR CONTROLS |  |  |
| - Inspect upper rod end seals for damage. | 41-59 step 6 |  |
| - Grease Swashplate bearings. | 41-53 |  |
| - TAIL ROTOR CONTROLS |  |  |
| - Inspect upper rod end seals for damage. | 42-31 step 4 |  |
| - HYDRAULIC SYSTEM |  |  |
| - Inspect rod end seals for damage. In case of damaged seals, remove control rods, disassemble and inspect for condition. | 43-32 thru 43-37 |  |
| - Bleed both reservoirs | 43-5 |  |
| - Inspect hydraulic system for condition. | 43-11 |  |
| - Do the leakage check of the hydraulic system. | 43-12 |  |
| - Functionally test hydraulic system | 43-14 |  |
| - LUBRICATION SYSTEM |  |  |
| - Remove chip detector, clean and test magnetic retaining force of each plug - Requires special weighted ball bearings. | 63-21, 02-14 63-23 |  |
| - STANDARD EQUIPMENT |  |  |
| - Inspect the following: |  |  |
| - Inspect windshield wiper system for condition and function | 702-3, 702-4 |  |
| - Inspect fire extinguishing system for condition and function. | 702-3, 702-4 |  |
| - Inspect hand fire extinguisher for security of attachment and proper charge |  |  |
| - Inspect first aid kit for completeness of contents and security of attachment |  |  |
| - Identification and other markings for legibility, and adhesive-backed decals for damage and proper adhesion |  |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - INSTRUMENT SYSTEM |  |  |
| - Inspect the following; |  |  |
| - Instrument panel decals for presence and readability |  |  |
| - Visible parts of pitot static system for condition | 91-23 |  |
| - Directional gyro for function | 91-49 |  |
| - Check the following; |  |  |
| - Pitot system for leakage | 91-21 step 1 |  |
| - Airspeed indicator for correct indication | 91-21 step 2 |  |
| - Altimeter for correct indication | 91-21 step 4 |  |
| - Static system for leakage | 91-21 step 3 |  |
| - Compensate compasses and inspect for condition | 91-46, 91-47, |  |
|  | 91-56 |  |
| - ELECTRICAL SYSTEMS |  |  |
| - Inspect ground leads and ground connections of fueling nozzle and fuselage/landing gear for condition. |  |  |
| - Inspect battery connector and cable for condition. | 92--65 step 1 |  |
| - EMERGENCY FLOAT SYSTEM MAX. TOW 2850 KG |  |  |
| - Inspect emergency float system for condition (floats installed) |  |  |
| - Check emergency float system for operational readiness. | 802--4 |  |
| NOTE: Circuit breaker EMER FLOATS must be set and toggle switch EMER FLOATS must be in ARMED position. If green indicator light is not on, refer to para 802--2, TROUBLESHOOTING |  |  |
| - Perform trial inflation. | Refer to |  |
|  | manufacturer CMM 25--60--97 |  |
| - P/R CSAS SYSTEM |  |  |
| - Inspect P/R CSAS for condition. | 821--4 |  |
|  |  | MECHANIC’S INITIALS |
| MAINTENANCE ACTION | REFERENCE |  |
| - INSTANTANEOUS VERTICAL SPEED INDICATOR |  |  |
| - Examine indicators |  |  |
| - EMERGENCY POWER SUPPLY -- STANDBY HORIZON |  |  |
| - Perform discharge check | 826--10 |  |
| - EMERGENCY FLOAT SYSTEM MAX. TOW 3350 KG |  |  |
| - Inspect emergency float system for condition. | 835--4, without |  |
|  | step 9 |  |
| - Perform trial inflation test. | Manufacturer’s Component Maintenance Manual CMM 25--63--04 |  |
| - SCAVENGE OIL FILTER SYSTEM |  |  |
| - Remove magnetic plugs of intermediate gearbox and tail rotor gearbox and test their magnetic retaining force - Requires special weighted ball bearings | 836--9, 02--14, 836--10 |  |
| - POST INSPECTION |  |  |
| - Ensure applicable servicing requirements have been carried out. |  |  |
| - Check oil level of transmission, hydraulic tank, engine and rail rotor gearbox prior to running helicopter. |  |  |
| - Complete a ground run at 100% NR to check for leaks and confirm system operation and parameters are within Flight Manual limitations. – Qualified pilot only. |  |  |
| - Replace or close all applicable trim panels, covers, access and inspection doors/covers as necessary. |  |  |
| - Perform Functional Check Flight if required per BK-117 Maintenance Manual or RFM as appropriate. |  |  |
| - Make an entry in the appropriate logbook (refer to the following page for wording of approval for return to service). |  |  |

Upon satisfactory completion of each inspection item, the mechanic shall initial the pertinent space as provided.  All mechanics shall sign and initial, in the space provided, at the end of the form.

At the completion of each inspection form, an A&P mechanic shall check the entire form for completion and then enter his/her signature, certificate type and number, and date in the space provided at the end of the form.  The same individual is also responsible for making the logbook entry shown at the conclusion of this inspection.

I certify that I have inspected and/or completed all the items initialed by me in accordance with Maritime Helicopters’ Eurocopter BO-117 Series Approved Aircraft Inspection Program and found them to be in an airworthy condition except as may be noted in the Aircraft Logbook.

| INITIALS | SIGNATURE | CERTIFICATE TYPE & NUMBER |
| --- | --- | --- |

I have reviewed this inspection form to verify that all items have been completed and have made an entry in the Aircraft Log Book.

I certify that this aircraft has been inspected in accordance with Maritime Helicopters’ Eurocopter BO-117 Series AAIP – A 1700 – 12 Month Inspection and was determined to be in an airworthy condition.

| Registration No.: | N | End |  | Aircraft Ending Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Log Page Number: |  |

| Signature: |  |  | Certificate Type & No: |  |
| --- | --- | --- | --- | --- |