| Registration No.: | N | Start |  | Aircraft Start Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Location: |  |

Reference:  Eurocopter B0-105 Series MM Chapter 101

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - GENERAL |  |  |
| - Logbook review for recorded discrepancies. Correct discrepancies on installed equipment. |  |  |
| - Ensure all applicable and required Special Inspections, Alert Service Bulletins (ASB’s), Engine Service Bulletins, and Airworthiness Directives (AD’s) are accomplished. |  |  |
| - Replace all life limited components that have completed published operating limits. | Eurocopter |  |
|  | BO-105 MP |  |
| - Overhaul all components that have completed published operating limits. | Eurocopter |  |
|  | BO-105 MP |  |
| - PLACARDS AND MARKINGS |  |  |
| - Examine the placards, decals, and markings. Make sure you can read them, they are applied correctly, and they are in agreement with the applicable configuration of your helicopter. |  |  |
| - CORROSION CONTROL |  |  |
| - Perform corrosion inspection in any area’s accessed during this AAIP inspection. |  |  |
| - MAIN TRASMISSION |  |  |
| - Inspect main transmission and attached parts for condition. | 11-3 |  |
|  | step 1. thru 16. |  |
| - Remove magnetic plug and inspect for deposits, clean and test for magnetic retaining force. | 11-17, 11-18, 11-19, 02-14 |  |
| - Inspect underside of rotor mast flange in the area of bores, including webs between the bores, for cracks, using a magnifying glass | AD 97 26-02 |  |
| - ROTOR BRAKE SYSTEM |  |  |
| - Inspect rotor brake system for condition | 12-3 |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - MAIN ROTOR SYSTEM |  |  |
| - Inspect main rotor system for condition | 13-3 |  |
| - Remove primary bolts and secondary bolts, clean and inspect for condition; crack inspection not necessary | 13-10, -11, -14, -16, -17, -18 |  |
| - MAIN ROTOR BLADE |  |  |
| - Inspect main rotor blade for condition | 14-2 or 14-3 |  |
| NOTE: Note following when inspecting erosion protective shells for debonding: When first-time debonds are found, or existing debonds have increased in area, the inspection shall be repeated after 50Fh. Only when an inspection has revealed no further debonding do the periodic inspection intervals re-apply |  |  |
| - FUSELAGE |  |  |
| - Inspect following frames for cracks: |  |  |
| - Upper corners of frames 7, 8, 9, forward and aft, and diagonal frame and frame 10. |  |  |
| - Lower frame segments 7 and 8. |  |  |
| NOTE: Stop-drill cracks. Repair any cracks in the upper area of frames 7 and 9 and diagonal frame within the next 50Fh. Repair any cracks in the upper area of frames 8 and 10 and in the lower area of frames 7 and 8 within the next 600Fh. Check for crack propagation every 200Fh. Repair cracks which show propagation within the next 50Fh. |  |  |
| - Inspect landing gear mounting brackets and surrounding fuselage area for condition | 21-3, step 10. |  |
| - Inspect transmission strut attach fittings and engine mount attach fittings for condition and inspect fuselage area adjacent to attach fittings for loose rivets and cracks | 21-3, step 5 and 7 |  |
| - Clean, grease and functionally check jettison device of cockpit doors. Renew lead wire seal. | 21-4 |  |
| - Ensure drain holes are not obstructed. | 21-3, step 4. |  |
| - TAILBOOM |  |  |
| - Inspect tail boom assembly for condition. | 31-2 |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - TAIL ROTOR DRIVE |  |  |
| - Inspect tail rotor drive for condition | 33-4, step 1 -- 7 |  |
| - Remove magnetic plugs of intermediate gearbox and tail rotor gearbox and test their magnetic retaining force | 32-9, 02-14 |  |
| - TAIL ROTOR |  |  |
| - Inspect tail rotor for condition | 33-4, step 1 -- 7 |  |
| - Remove and inspect blade mounting forks for condition. | 33-14 thru 33-18 |  |
| EFFECTIVITY: Tail rotor blade mounting forks made from aluminum alloy P/N 105--31711, 105--31722 |  |  |
| - Remove bellcrank assy and inspect for condition | 33-9 thru 33-12 |  |
| - Check ball bearings of sliding sleeve for proper running and absence of play. | 33-27, step 14 |  |
| - Inspect balance weight and control lever for condition. | 33-12C |  |
| NOTE: Enter AD 2013-16-13 as separate entry in logbook and sign off |  |  |
| - TAIL ROTOR BLADE |  |  |
| - Inspect tail rotor blades for condition. | 34-2 |  |
| - MAIN ROTOR CONTROLS |  |  |
| - Inspect all flight control components for condition | 41-3 |  |
| - Check swash plate bearing for proper running. | 41-73 |  |
| - Check rigging of collective control forces. | 102-16, step 9. |  |
| NOTE: Hydraulic mule required. |  |  |
| - Clean swash plate sliding surface on support tube. Move swash plate to uppermost position to expose sliding surface. Use cleaning solvent (CM 201, 202 or 217) |  |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - TAIL ROTOR CONTROLS |  |  |
| - Visually inspect the seals on rod ends and clevis for damage on the following; |  |  |
| - Control rod in the tail boom | 42-3 |  |
| - Control rod in vertical fin (upper clevis) |  |  |
| - Visual inspect sealing of blind hole in fork end for damage in control rod in vertical fin. |  |  |
| - HYDRAULIC SYSTEM |  |  |
| - Inspect switchover control linkage for play. | 43-17 or 43-17A |  |
| - Inspect hydraulic system for condition. | 43-11 |  |
| - Spray-coat N2-lever (droop-compensation) with corrosion preventive compound (CM 508). |  |  |
| - Check filter contamination indicator (lateral indicator pin configuration). | 43-14 |  |
| - Functionally test hydraulic system | 43-15 |  |
| - Perform ground run check of selector valve setting. | 43-16 or 43-16A |  |
| - Inspect system 1 selector valve for corrosion. | 3-13 |  |
| - LANDING GEAR |  |  |
| - Inspect landing gear for condition. | 51-2 |  |
| - ENGINE |  |  |
| - Remove anti-icing valve actuators and inspect mounting brackets for condition. | 61-37, 61-38, 61-39 |  |
| - FUEL SYSTEM |  |  |
| - Inspect fuel lines in the engine compartment for condition | 62-3 |  |
| - Check fuel differential pressure switch for function. | 62-39 |  |
| - Perform functional test and leakage test of fuel shutoff valves. | 92-82, 92-83 |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - LUBRICATION SYSTEM |  |  |
| - Inspect lubrication system for condition | 63-3 |  |
| - Remove oil drain plugs of oil tank and test the magnetic retaining force of their respective magnetic plugs. | 63-4, step 1.b, 02-14 |  |
| - Visually inspect baffle plate, located between oil cooler and fan, and attaching clamps for cracks |  |  |
| EFFECTIVITY: Ram air ventilation not installed: |  |  |
| - FIREWALLS |  |  |
| - Inspect fire walls for condition. | 64-2 |  |
| - ENGINE OPERATION AND CONTROL |  |  |
| - Inspect engine operation and control linkages for condition. | 65-3 |  |
| - STANDARD EQUIPMENT |  |  |
| - Inspect the following: |  |  |
| - Pilot and copilot seats for condition | 701-2 |  |
| - Cabin equipment for condition | 702-2 |  |
| - Cabin ventilation for condition, check for function | 704-2 |  |
| - Identification and other markings for legibility, and adhesive-backed decals for damage and proper adhesion |  |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - INSTRUMENT SYSTEM |  |  |
| - Ventilate dual torque indication system. | 91-57 |  |
| - Inspect instruments for condition. | 91-4 |  |
| - Clean and drain pitot-static system, inspect for condition and check for function | 91-9 thru 91-12 |  |
| - Inspect magnetic compass for condition and function and compensate magnetic compass - Requires master compass | 91-24 thru 91-26 |  |
| - Calibrate TOT indication system - Barfield Required | 91-45 |  |
| - Inspect instrument panel, center panel and overhead panel for presence, legibility and security of labels and placards. |  |  |
| NOTE: Replace missing placards and rebond separated placards |  |  |
| - ELECTRICAL SYSTEMS |  |  |
| - Inspect battery bonding jumper for tight installation and unimpaired surface protection |  |  |
| - Check voltage regulator setting - Requires calibrated Volt Meter | 92-17 |  |
| - Inspect socket of anti-collision light for condition. | 92-116 |  |
| - Inspect battery--temperature warning. | 92--34E |  |
| - Remove chip detector, check, clean, test for function and test magnetic retaining force - Weighted ball bearings required. | 845-4 thru 6, 02-14 |  |
| - MAIN ROTOR BLADE FOLDING SYSTEM |  |  |
| - Remove locking lever assembly, clean and inspect for condition (crack inspection not required) | 854-3, step 1. |  |
| - Remove locking lever assembly. |  |  |
| - Clean and inspect nickel-plated bolts for condition, and provide with corrosion protection (crack inspection not required) | 854-3, step 1.a 854-4, step 2.f |  |
| - Clean and inspect solid film lubricant-coated bolts for condition (crack inspection not required). Coat bolt with lubricant (CM146} | 854-3, step 1.b 854-4, step 2.g |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - CONTINUOUS IGNITION SYSTEM |  |  |
| - Check spark igniters for condition | 857-4 |  |
| - Visually inspect noise suppressor and accessible parts of wiring for condition. | 857-4 |  |
| - POST INSPECTION |  |  |
| - Ensure applicable servicing requirements have been carried out. |  |  |
| - Check oil level of transmission, hydraulic tank, engine and rail rotor gearbox prior to running helicopter. |  |  |
| - Complete a ground run at 100% NR to check for leaks and confirm system operation and parameters are within Flight Manual limitations. – Qualified pilot only. |  |  |
| - Replace or close all applicable trim panels, covers, access and inspection doors/covers as necessary. |  |  |
| - Perform Functional Check Flight if required per Bell 206L Maintenance Manual or RFM as appropriate. |  |  |
| - Make an entry in the appropriate logbook (refer to the following page for wording of approval for return to service). |  |  |

Upon satisfactory completion of each inspection item, the mechanic shall initial the pertinent space as provided.  All mechanics shall sign and initial, in the space provided, at the end of the form.

At the completion of each inspection form, an A&P mechanic shall check the entire form for completion and then enter his/her signature, certificate type and number, and date in the space provided at the end of the form.  The same individual is also responsible for making the logbook entry shown at the conclusion of this inspection.

I certify that I have inspected and/or completed all the items initialed by me in accordance with Maritime Helicopters’ Eurocopter BO-105 Series Approved Aircraft Inspection Program and found them to be in an airworthy condition except as may be noted in the Aircraft Logbook.

| INITIALS | SIGNATURE | CERTIFICATE TYPE & NUMBER |
| --- | --- | --- |

I have reviewed this inspection form to verify that all items have been completed and have made an entry in the Aircraft Log Book.

I certify that this aircraft has been inspected in accordance with Maritime Helicopters’ Eurocopter BO-105 Series AAIP – A 6090 – 6000 Hour / 96 Month Inspection and was determined to be in an airworthy condition.

| Registration No.: | N | End |  | Aircraft Ending Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Log Page Number: |  |

| Signature: |  |  | Certificate Type & No: |  |
| --- | --- | --- | --- | --- |