Maritime MTL’s

| General | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Aircraft Registration | 36 Month |  | 36 Month aircraft registration with FAA | FAA.gov web site and FAR 47 |

NOTE 1: this inspection was created by Bristow Alaska Inc. to check against the aircraft records.

| Chapter 08 Weight and Balance | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Weigh Aircraft | 24 Month |  | Weigh Aircraft | AMD Requirement Bristow Alaska GMM Appendix S |

| Chapter 12 Servicing | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Main Rotor Thixo Grease Lubrication | 25 Hour |  | Lubricate main rotor hub with Thixo Grease | IL GEN-12-125 |
| Transmission Oil | 200 Hour |  | Drain and re-service transmission oil (See phases 1,3,5) | BHT 206 MM Ch. 12 (NOTE 1) |
| Transmission Oil Filter Element | 200 Hour |  | Replace Filter Element (See phases 1,3,5) | BHT 206 MM Ch. 12 (NOTE 1) |
| Transmission Oil pump inlet screen | 200 Hour |  | clean and inspect (See phases 1,3,5) | BHT 206 MM Ch. 12 |
| Tail Rotor Gearbox Oil | 100 Hour |  | Drain and re-service tail rotor gearbox oil (See all phases) | BHT 206 MM Ch. 12 (NOTE 1,2) |
| Engine gearbox scavenge oil filter | 150 Hour |  | Remove, inspect, clean and reinstall the oil filter. (See engine inspection) | RR-250 C-30 OMM 72-00-00 Table 603 |
| Engine oil | 600 Hour/12 Month |  | Drain and re-service engine oil. | RR-250 C-30 OMM 72-00-00 Table 603 |
| Sliding Adapter Splines, tail rotor driveshaft (four places) | 200 Hour |  | Remove, clean, inspect, lubricate, and install splined adapters (see phases 2,4,6) | BHT 206 MM Ch. 12 (NOTE 1,2) |
| Hydraulic System Fluid | 300 Hour |  | Drain and re-service Hydraulic fluid (see phases 3,6) | BHT 206 MM Ch. 12 (NOTE 2) |
| Hydraulic Filters | 300 Hour |  | Remove and clean hydraulic filters (see phases 3,6) | BHT 206 MM Ch. 12 (NOTE 3) |
| Hydraulic Pump Drive | 300 Hour |  | Remove pump and lubricate shaft splines (see phases 3,6) | BHT 206 MM Ch. 12 (NOTE 1) |
| N1 and N2 Tach-generator Drive | 600 Hour |  | Remove tach-generators and lubricate shaft splines | BHT 206 MM Ch. 12 (NOTE 2) |
| NR Tach-generator Drive (output splines for hydraulic pump) | 300 Hour |  | Remove NR tach-generator and lubricate output splines (see phases 3,6) | BHT 206 MM Ch. 12 (NOTE 1) |
| Drive, NR Tach-generator (internal input drive splines and driveshaft from oil pump) | 600 Hour |  | Remove NR tach-generator and lubricate input drive splines (see phase 6) | BHT 206 MM Ch. 12 (NOTE 1) |
| Airframe Fuel Filter | 300 Hour |  | Replace airframe fuel filter (see phases 1,4) | BHT 206 MM Ch. 12 |

NOTE 1: 12 Month inspection not performed based on Bristow previous experience.

NOTE 2: Time reduced based on Bristow previous experience.

NOTE 3: Filter element may be cleaned only three times.

| Chapter 24 Electrical Power | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Starter Generator | 200 Hour |  | Brush inspection (See starter/generator checklist) | Goodrich MM 23700, Bristow Alaska GMM Appendix A |
| Starter Generator | 400 Hour | 1200 Hour | Brush Change (See starter/generator checklist) | Goodrich MM 23700, Bristow Alaska GMM Appendix A (NOTE 1) |
| Aircraft Battery, (NiCad) | 6 Month |  | Recondition and deep cycle | SAFT or Marathon Maintenance Manual |
| Aircraft Battery, (Lead Acid) | 12 Month | 4 year | 12 month capacitance check | Concorde CMM 24-30-71 (NOTE 2) |
| Airframe Battery Connector | 12 Month |  | 12 month serviceability check | Marathon Maintenance Manual Inspection Section |

NOTE 1: Overhaul time increase based on Bristow previous experience. Long life brushes (P/N 23081-1834) with more than 50% life do not require replacement

NOTE 2: Initial check at 12 months after initial installation. As long as the capacity is above 90%, subsequent capacity checks are every 6 months in service. If the capacity is between 85 and 90%, subsequent capacity checks are every 3 months in service. If capacity is less than 85%, remove battery from service.

| Chapter 25 Equipment/ Furnishings | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Cargo Hook | 200 Hour | 60 Month | Visual Inspection (See phase 1,3,5) | Onboard Systems ICA |
| Personal Locator Beacon (PLB) | 6 Month |  | Visual inspection and BIT test | TECHTEST MM |
| Personal Locator Beacon (PLB) | 24 Month |  | Registration renewal | TECHTEST MM |
| Personal Locator Beacon (PLB) | 60 Month |  | Replace battery | TECHTEST MM |
| Acoustic Beacon | 6 Month | 72 Month (Replace) | 6 month serviceability check | Manufactures maintenance manual |
| Fire Extinguishers | 1 Month |  | 1 month visual inspection | NFPA 10 |
| Fire Extinguishers | 12 Month |  | Remove and replace for 12 month serviceability check | NFPA 10 |
| Fire Extinguishers | 72 Month |  | Remove and replace for 72 month internal inspection | NFPA 10 |
| Fire Extinguishers | 144 Month |  | Hydrostatic test | NFPA 10 |
| ELT | 1 Month |  | 1 month self-test | Artex manual |
| ELT | 12 Month |  | 12 month serviceably check (See avionics Inspection) | Artex manual, FAR 91.207(d) |
| ELT | 24 Month |  | Registration renewal | Bristow Alaska GMM |
| ELT | 60 Month |  | Replace battery | Artex manual |
| ELT Remote Switch | 60 Month |  | Replace battery | Artex manual |
| Life Vests | 24 Month |  | 24 month serviceably check | Switlik manual |
| First Aid Kit | 12 Month |  | Inspect kit for proper contents | Bristow Alaska GMM |
| Survival Kit | 12 Month |  | Inspect kit for proper contents | Bristow Alaska GMM |
| Tanis Heater Kit | 12 Month |  | 12 month serviceability check | Tanis ICA TCA0003 |
| Cabin Heater System | 12 Month |  | 12 month serviceability check | Air Comm Corporation ICA 206H-227M |

| Chapter 28 Fuel System | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Low Fuel Caution System | 12 Month |  | 12 month serviceability check | BHT 206 MM Ch. 5-33 Bristow Alaska GMM Appendix S |
| Fuel System Calibration Check | 12 Month |  | 12 month serviceability check | BHT 206 MM Ch. 5-33 Bristow Alaska GMM Appendix S |
| Fuel System Cleaning | 12 Month |  | 12 month serviceability check | BHT 206 MM Ch. 5-33,44 Bristow Alaska GMM Appendix S |

| Chapter 32 Landing Gear | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| DART Bear Paws | 300 Hour |  | 300 Hour inspection (See phase 3,6) | DART ICA-206-559 (NOTE 1) |
| Cross tubes | 200 Hour |  | 200 Hour inspection (See phase 1,3,5) | Aeronautical Accessories Inc. ICA 01149, DART ICA-D206-667 (NOTE 1,2) |
| DART Cross tubes | 24 Month |  | 24 Month inspection | DART ICA-D206-667 |
| Flite Steps | 300 Hour |  | 300 Hour inspection (See phase 1,4) | Aeronautical Accessories Inc. ICA 01161, DART ICA-D206-628 (NOTE 1) |

NOTE 1: 12 month inspection not required.

NOTE 2: Time reduced based on Bristow previous experience.

| Chapter 34 Navigation | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Avionics Inspection | 12 Month |  | 12 month function check of avionics systems | Bristow Alaska GMM Appendix S |
| VOR | 12 Month |  | 12 month serviceability check (See Avionics Inspection) | FAR 91.171 |
| Standby Compass | As required see section 2.05 Conditional Inspections |  | Compass calibration check | AC 43.13-1B 12-37 |
| Transponder | 24 Month |  | 24 month systems serviceability check | FAR 91.413 and 43 Appendix F |
| Pitot Static Check | 24 Month |  | 24 month systems serviceability check | AMD Requirement FAR 43 Appendix E (NOTE 1) |

NOTE 1: Under certain circumstances where maintenance has been performed which could affect the system, the integrated system test must be accomplished.

| Chapter 53 Fuselage | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Tailboom | 50 Hour |  | 50 Hour visual inspection | AD 96-18-05 (NOTE 1) |
| Tailboom | 100 Hour |  | 100 Hour visual inspection for cracks (See all phases) | BHT 206 MM Ch. 5-19, ASB 206L-09-158 Part 1, Latest revision |
| Tailboom | 1200 Hour |  | 1200 Hour visual inspection | AD 2002-25-07 C/W Part IV of ASB 206L-99-115 Latest revision (NOTE 2) |

NOTE 1:	Applies to tailboom P/N 206-033-004-3/-11/-45/103

NOTE 2:	ASB 206L-99-115 Applies to tailboom P/N 206-033-004-ALL

| Chapter 62 Main Rotor | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Main Rotor Assembly | 12 Month |  | Dynamic Balance | Chadwick manual or equivalent |
| Main Rotor Hub | 1250 Hour | 2400 Hour (NOTE 1) | In shop inspection | BHT 206 MM Ch. 5-58 |

NOTE 1:	1200 Hour overhaul for main rotor hub assemblies pre TB 206L-91-159.

| Chapter 63 Main Rotor Drive | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Rotor Brake Caliper | 600 Hour | 1800 Hour | Out of aircraft inspection | Northwest Dynamics Inc. |
| Rotor Brake Disk | 600 Hour | 3000 Hour | Out of aircraft inspection | Northwest Dynamics Inc. |
| Mast | 12 Month |  | 12 Month visual inspection | BHT 206 MM Ch. 5-34 (NOTE 1) |
| Mast Assembly | 1550 Hour | 3000 Hour | In shop inspection | BHT 206 MM Ch. 5-40 BHT 206 MM Ch. 5-58 |
| Swashplate and Support Assembly |  | 4800 Hour | Overhaul | BHT 206 MM Ch. 5-58 |
| Freewheel Assembly | 60 Month | 3000 Hour | 1500 Hour/60 Month in shop inspection 3000 Hour overhaul | BHT 206 MM Ch. 5-36 BHT 206 MM Ch. 5-58 |
| Transmission Assembly | 1550 Hour |  | In shop inspection | BHT 206 MM Ch. 5-40 Pre TB 206L-04-213 |
| Transmission Assembly | 2250 Hour/60 month | 4500 Hour | 2250 Hour/60 month In shop teardown | BHT 206 MM Ch. 5-40 Post TB 206L-04-213 |
| Main Drive Shaft Keflex |  | 2500 Hour | Overhaul | Kamatics Corporation (NOTE 2) |

NOTE 1:	ASB 206L-83-30 Incorporated into MM.

NOTE 2:	Bell 206L-4 drive shaft P/N: 206-340-300-ALL must be overhauled at 5000 Hours of operation and every 2500 Hours of operation thereafter. Overhauls to be accomplished by Kamatics Corporation only.

| Chapter 64 Tail Rotor | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Tail Rotor Balance | 300 Hour |  | 300 Hour Dynamic Balance | Chadwick manual or equivalent |
| Tail Rotor Hub Assembly |  | 2500 Hour | Overhaul | BHT 206 MM Ch. 5-58 (NOTE 1) |

NOTE 1:	2400 Hour overhaul for tail rotor hub assembly P/N 206-011-810-015

| Chapter 65 Tail Rotor Drive | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| #2 Drive Shaft (Oil Cooler Blower) | 300 Hour |  | 300 Hour clean and vibration check of the oil cooler blower bearings | BHT 206 MM Ch. 12 Bristow Alaska GMM Appendix S |
| Tail Rotor Gearbox | 3000 Hour | 6000 Hour | In shop inspection | BHT 206 MM Ch. 5-42 BHT 206 MM Ch. 5-58 |

| Chapter 67 Flight Controls | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Tail Rotor Control Tube | 1200 Hour |  | 1200 Hour visual inspection | BHT 206 MM Ch. 5-30 |
| Cyclic Control Stick | 1200 Hour |  | 1200 Hour visual inspection | BHT 206 MM Ch. 5-30 |
| Servo Actuator |  | 3600 Hour | Overhaul | BHT 206 MM Ch. 5-58 |
| Flight Control Bolts | 24 Month |  | 24 Month visual inspection | BHT 206 MM Ch. 5-35 (NOTE 1) |

NOTE 1:	ASB 206L-89-61 Incorporated into MM.

| Chapter 72 Engine | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Turbine Outlet Temperature Check | 300 Hour |  | 300 Hour inspect and test (See engine Inspection) | BHT 206 MM Ch. 5-29 |
| Compressor 250 C-30P | 2000 Hour |  | Inspection | RR-250 C-30 OMM 72-30-00 |
| Combustion Liner | 2000 Hour |  | Inspection | RR-250 C-30 OMM 72-40-00 |
| Turbine Assembly |  | 2000 Hour | Overhaul | RR-250 C-30 OMM 72-50-00 |
| Gearbox Assembly |  | 20000 Hour | Overhaul | RR-250 C-30 OMM 72-60-00 |

| Chapter 73 Engine Fuel Control | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Fuel Control (BENDIX) |  | 2500 Hour | Overhaul | RR-250 C-30 OMM 73-20-02 |
| Governor |  | 2000 Hour | Overhaul | RR-250 C-30 OMM 73-20-01 |
| Fuel Pump Sunstrand |  | 1500 Hour | Overhaul | RR-250 C-30 OMM 73-10-01 |
| Fuel Pump TRW/Argo Tech |  | 3000 Hour | Overhaul | RR-250 C-30 OMM 73-10-01 |
| Fuel Pump -CECO |  | 3500 Hour | Overhaul | RR-250 C-30 OMM 73-10-01 |
| Fuel Nozzle | 150 Hour | 2000 Hour | 150 Hour clean and inspect | RR-250 C-30 OMM 73-10-03 |

| Chapter 75 Engine Air | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| Compressor Bleed Valve |  | 1500 Hour | Overhaul | RR-250 C-30 OMM 75-10-02 |

| Chapter 79 Oil | Inspection Interval | Overhaul Interval | Task Required | Reference |
| --- | --- | --- | --- | --- |
| NiCad External Oil Filter | 600 Hour |  | 600 Hour bypass check | RR CSL-3116 |

## 2.03 Required Torque Check

| Torque Check | Hours since new or installed |
| --- | --- |
| Main rotor mast nut | 1-8 Hours |
| Main rotor pillow block bolts. | 3-8 Hours |
| Tail rotor drive shaft disk pack coupling | 10-25 Hours |
| Tail rotor gearbox mount nuts | 10-25 Hours |
| Swashplate tilt friction | 10-25 Hours |
| Engine mount attachment hardware at engine and airframe points | 90-110 Hours |
| Transmission top case stud nuts | 90-110 Hours |
| Tailboom attachment bolts | 90-110 Hours |

## 2.04 After Installation Check

| After Installation Check | Hours since new, overhauled, or installed |
| --- | --- |
| Engine oil system (NOTE 1) | 50-75 Hours |
| After transmission installation (NOTE 2) | 90-110 Hours |

NOTE 1:	Oil system contamination inspection to be complied with after the first 50 hours of operation of a new, overhauled, or repaired engine. See RR OMM Ch. 72-00-00

NOTE 2:	Replace oil and filter element. Inspect removed element for metal particles. If particles are found, investigate to determine cause.

## 2.05 Conditional Inspections

Conditional Inspection

A compass swing must be performed on the following occasions: - Whenever a new compass is installed. - When the accuracy of the compass is suspected. - After any cockpit modification or major replacement involving ferrous metal. - Whenever a compass has been subjected to a shock; for example, after a hard landing or severe turbulence. - After aircraft has passed through a severe electrical storm. - After lighting strike. - Whenever a change is made to the electrical system. - Whenever a change of cargo is likely to affect the compass. - When an aircraft operation is changed to a different geographic location (e.g., Miami, Florida to Fairbanks, Alaska) with a major change in magnetic deviation. - After aircraft has been parked on one heading for over a year. - When flux valves are replaced.