| Registration No.: | N | Start |  | Aircraft Start Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Start Total Cycles: |  |

Reference:  BHT 206L Series MM-1 Chapter 4, 5, 12

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - GENERAL |  |  |
| - Logbook review for recorded discrepancies. Correct discrepancies on installed equipment. |  |  |
| - Ensure all applicable and required Special Inspections, Alert Service Bulletins (ASB’s), Engine Service Bulletins, and Airworthiness Directives (AD’s) are accomplished. |  |  |
| - Replace all life limited components that have completed published operating limits. | Bell 206L MP |  |
| - Overhaul all components that have completed published operating limits. | Bell 206L MP |  |
| - LUBRICATION |  |  |
| - Purge lubricate the following Main Rotor Components: | Table 12-5 |  |
| - Pitch horn trunnion bearing (four places) |  |  |
| - Pillow block/trunnion bearing (two places) |  |  |
| - Grip bearing (two places) |  |  |
| - Swashplate duplex bearing |  |  |
| - Tail rotor trunnion bearing |  |  |
| - MAIN ROTOR SYSTEM |  |  |
| - Examine the placards, decals ad markings. Make sure they are readable, correctly applied and in agreement with the applicable configuration of your helicopter. | Chp 11 |  |
| - Verify main rotor flap restraint for freedom of movement, and for condition and security. | Chp 62 |  |
| - Examine static stops and mast for evidence of mast bumping. |  |  |
| - Remove main rotor blade leading edge erosion protection tape if installed. |  |  |
| - Examine main rotor blades for cleaniness and overall condition. |  |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - Examine blades for cracks, corrosion and de-bonding of doublers. |  |  |
| - Examine leading edge of blades for corrosion and erosion. Restore as necessary. |  |  |
| - Examine the tip weight cap for corrosion, cracks, erosion, and deformation. |  |  |
| - If applicable, reinstall main rotor blade leading edge erosion protection tape |  |  |
| - Examine the following main rotor hub assembly components for condition and security |  |  |
| - Mast nut |  |  |
| - Trunnion |  |  |
| - Pillow blocks |  |  |
| - Grips |  |  |
| - Pitch horns |  |  |
| - Static stops |  |  |
| - Examine pitch horn trunnion bearings for evidence of wear, damage and for security. |  |  |
| - Torque check pillow block retention hardware 84 to 107 inch-pounds. Examine for misalignment of anti-slippage marks (ASB 206L-97-108). |  |  |
| - Examine main rotor yoke for corrosion or mechanical damage as follows: |  |  |
| - Examine fillet radius of both yoke spindles inboard of wear sleeves for evidence of corrosion. If corrosion is visible, remove with fine abrasive pad (C-407). If corrosion cannot be removed with pads, or is extensive, disassemble hub for inspection and repair. |  |  |
| NOTE If the main rotor hub must be disassemble, omit the remaining steps. |  |  |
| - Examine entire yoke centre section (inboard and outboard surface) for damage. |  |  |
| - Touch up all repaired areas or areas of bare metal in accordance with applicable main rotor hu inspection and repair instructions. |  |  |
| - Ensure no sealant voids are visible around shields or wear sleeve, strap fittings and grip closure. Recoat with sealant (C-308) as required. ON main rotor hub assemblies up to and including P/N 206-011-100-025, coat the remaining exposed fillet radius and repaired surfaces with a film of corrosion preventive compound (C-101). |  |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - TAIL ROTOR ASSEMBLY |  |  |
| - Examine the placards, decals ad markings. Make sure they are readable, correctly applied and in agreement with the applicable configuration of your helicopter. | Chp 11 |  |
| - Clean tail rotor blades to maintain visibility of warning stripes. | Chp 64 |  |
| - Examine the following tail rotor hub assembly components for condition and security of attachment: | BHT-206L-CR&O |  |
| - Yoke |  |  |
| - Pitch horns |  |  |
| - Trunnion and trunnion caps |  |  |
| - Counterweight hardware |  |  |
| - Examine tail rotor blades as follows: | Chp 64 |  |
| - Tip block rivets for damage, corrosion, erosion and looseness |  |  |
| - Skin for bulges, nicks, dents scratches or other damage. |  |  |
| - Deformed or cracked skin in the area of the chord wise weights. |  |  |
| - Leading edge for erosion, nicks, dents or scratches. |  |  |
| - Bonding joints of blades skin mating area for suspected voids or cracks. Pay particular attention in the area of tip block and doublers. |  |  |
| - Feathering bearings for looseness and cracks in uniball. |  |  |
| - Examine the following tail rotor hub and blade assembly attaching components for condition and security: |  |  |
| - Spacer |  |  |
| - Rubber bumper |  |  |
| - Static stop |  |  |
| - Nut |  |  |
| - Balance wheel |  |  |

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - AIRFRAME (FORWARD DECK) |  |  |
| - Examine the placards, decals ad markings. Make sure they are readable, correctly applied and in agreement with the applicable configuration of your helicopter. | Chp 11 |  |
| - Open and examine forward fairing for condition and security. | Chp 71 |  |
| - Remove and examine transmission cowling and security. |  |  |
| - Examine all fairing and cowling latches and/or fasteners for condition. |  |  |
| - Examine top deck for condition, leaks and security. |  |  |
| - Clean transmission deck. | Chp 53 |  |
| - Examine air induction cowling and plenum for damage, obstructions and cleanliness. Examine intake fairing windows for damage and cleanliness. |  |  |
| - Examine inlet screen (if applicable) for missing or damaged wires and rivets. |  |  |
| - Examine forward deck drain9s) to ensure absence of obstruction. |  |  |
| - Examine antennas for condition and security. |  |  |
| - CONTROLS |  |  |
| - Examine the following flight control components for signs of interference, corrosion, wear, mechanical damage and security: | Chp 67 |  |
| - Control tubes |  |  |
| - Links |  |  |
| - Rod and bearings |  |  |
| - Bellcranks |  |  |
| - Bellcrank supports |  |  |
| - Attaching bolts and nuts |  |  |
| - Inspect main rotor pitch links with a 3X magnifying glass for damage, corrosion and cracks. Giv particular attention to swaged ends at jam nut or inserts. |  |  |
| - Examine N2 control linkages on forward deck for condition and security. Pay particular attention to the jackshaft assembly for damage or evidence or twisting. |  |  |

| - POST INSPECTION |  |  |
| --- | --- | --- |
| - Inspect for operation and leaks. |  |  |
| - Replace or close all applicable trim panels, covers, access and inspection doors/covers as necessary. |  |  |
| - Perform Functional Check Flight if required per Bell 206L Series Maintenance Manual or RFM as appropriate. |  |  |
| - Make an entry in the appropriate logbook (refer to the following page for wording of approval for return to service). |  |  |

Upon satisfactory completion of each inspection item, the mechanic shall initial the pertinent space as provided.  All mechanics shall sign and initial, in the space provided, at the end of the form.

At the completion of each inspection form, an A&P mechanic shall check the entire form for completion and then enter his/her signature, certificate type and number, and date in the space provided at the end of the form.  The same individual is also responsible for making the logbook entry shown at the conclusion of this inspection.

I certify that I have inspected all the items initialed by me in accordance with Maritime Helicopters’ Bell 206L Series Approved Aircraft Inspection Program and found them to be in an airworthy condition except as may be noted in the Aircraft Logbook.

| INITIALS | SIGNATURE | CERTIFICATE TYPE & NUMBER |
| --- | --- | --- |

I have reviewed this inspection form to verify that all items have been completed and have made an entry in the Aircraft Log Book.

I certify that this aircraft has been inspected in accordance with Maritime Helicopters’ Bell 206L Series AAIP – A0051 – 50hr Inspection and was determined to be in an airworthy condition.

| Registration No.: | N | End |  | Aircraft Ending Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Ending Total Cycles: |  |

| Signature: |  |  | Certificate Type & No: |  |
| --- | --- | --- | --- | --- |