| Registration No.: | N | Start |  | Aircraft Start Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Start Total Cycles: |  |

Reference:  BHT 206L Series MM-1 Chapter 4, 5, 12

|  |  | MECHANIC’S INITIALS |
| --- | --- | --- |
| MAINTENANCE ACTION | REFERENCE |  |
| - GENERAL |  |  |
| - Logbook review for recorded discrepancies. Correct discrepancies on installed equipment. |  |  |
| - Ensure all applicable and required Special Inspections, Alert Service Bulletins (ASB’s), Engine Service Bulletins, and Airworthiness Directives (AD’s) are accomplished. |  |  |
| - Replace all life limited components that have completed published operating limits. | Bell 206L MP |  |
| - Overhaul all components that have completed published operating limits. | Bell 206L MP |  |
| - LUBRICATION |  |  |
| - Examine the placards, decals and markings. Make sure they are readable, correctly applied and in agreement with the applicable configuration of your helicopter. | BHT MM Ch. 11 |  |
| - Inspect passenger and pilot compartment of condition of interior panels, seats for tears, deterioration and security of attachment. Inspect condition and operation of heater nozzles and valves. |  |  |
| - Inspect cabin doors, hinges for wear and security, door seals and windows for condition and latches for proper operation. |  |  |
| - Inspect battery compartment for condition, battery and electrical relays and wiring for condition and security. |  |  |
| - Inspect strobe lights for security and operation, lens for cleanliness, clean and replace as necessary. |  |  |
| - Inspect interior and exterior lights for operation. |  |  |
| - Inspect landing gear for obvious damage, cross tubes for excessive deflection, skid tubes and shoes for security of attachment and wear. If other accessories, such as flight steps, tundra pads, or baskets are installed, check for condition and security. |  |  |
| - Inspect baggage compartment for obvious damage, loose equipment properly stowed, and cargo tie downs in good condition. |  |  |
| - Remove forward upper cowling, inspect hydraulic actuators and flexible lines for leakage and chafing, and inspect bearings for wear and security of attachment. |  |  |
| - Inspect all control tubes, bell cranks, and bearings from servos to swashplate for obvious damage, wear and security of attachment. Inspect servo pivot bolts for freedom of rotation and security. |  |  |
| - Inspect transmission mounts, up and down stops, arm and flexure assemblies for evidence of damage and security of attachment. Inspect elastomeric bearings in four support assemblies, transmission restraint in four link assemblies for evidence of cuts, flex cracks and bond separation. |  |  |
| - Inspect transmission deck and servo area for loose bolts, broken or loose connectors, broken or missing safeties, leakage, and fluid lines for chafing and security of attachment. Clean area as needed. |  |  |
| - Inspect swashplate for condition, visible damage and security. Check base of swashplate support for cracks. Inspect drive link and collar for wear. Lubricate with Mobil 28 grease. Clean grease slung from swashplate off of collective sleeve and collective lever. |  |  |
| - Reinstall forward upper cowling. |  |  |
| - Check mast for obvious damage, nicks and corrosion. |  |  |
| - Inspect main rotor pitch links for damage, security of attachment, corrosion and cracks. Pay particular attention to swaged ends at jam nut or insert fraying surface. |  |  |
| - Inspect main rotor hub for damage and condition. Inspect blade bolts, latch bolts, grips, pillow block, and mast nut for damage and security of attachment. Inspect pitch horn trunnion bearing for wear. Clean, then inspect with a bright light, the main rotor yoke web inboard and outboard between pillow block bores, tooling holes and lower flange for damage or cracks. Inspect yoke fillet radii for evidence of corrosion. |  |  |
| - Inspect main rotor blades for obvious damage, corrosion, separation of laminate fingers (use coin tap method), check trim tab for condition and cracks at radii and tip cap for security. Check for cord wise cracks from trailing edge. |  |  |
| - If required and conditions permit, wipe main rotor hub and blades with solvent to remove oil and grease. Wash main rotor hub and blades with mild solution (ratio 4 water, 1 cleaner) or detergent soap. Rinse with fresh water and wipe dry with clean cloth. |  |  |
| - Inspect particle separator inlets for obstructions and damage. Inspect engine intake bay through cowling inspection windows for foreign objects. Check snow deflectors for security when installed. |  |  |
| - Inspect rotor brake system for security of attachment, condition, leakage and chafing of lines. |  |  |
| - Inspect the engine for obvious loose bolts, broken or loose connections, security of mounting accessories, and broken or missing safeties. Check accessible areas for obvious damage and evidence of fuel and oil leakage. Check B-nuts for presence and alignment of torque stripes. B-nuts with missing torque stripes must be loosened and retightened before application of new torque stripes |  |  |
| - Inspect ignition leads and electrical harness for condition. |  |  |
| - Inspect engine mounts for damage, corrosion and security. |  |  |
| - Inspect starter generator for condition of leads and attachment clamp for security. Check condition of cooling duct. |  |  |
| - Inspect turbine support assemblies, outer combustion case, and engine exhaust ducts for condition of welded joints, cracks and buckling, and exhaust duct for proper installation. |  |  |
| - Inspect the anti-icing, bleed air, and overspeed solenoid valves for loose, chafed frayed or broken wires; loose connections and security of attachment. |  |  |
| - Remove cowling from oil tank and cooler, inspect oil cooler blower hanger bearings for grease leakage and evidence of overheating, blower for obstructions and foreign matter. Check oil cooler and oil tank mount brackets for condition. Clean as needed. |  |  |
| - Remove tail rotor drive shaft cover. Inspect tail boom, and tail rotor drive shaft cover for obvious damage and security of attachment. Look closely at seams, joints, edges and fasteners. Check safeties at all hanger assemblies, check bearings for signs of overheating |  |  |
| - Remove tail rotor gearbox upper cowling. Inspect tail rotor hub assembly components for condition and security. |  |  |
| - Reinstall removed cowlings. |  |  |
| - Inspect tail rotor blades as follows: Tip blocks for damage, corrosion, erosion, and looseness. Skin for bulges, nicks, dents, scratches, or other damage. Deformed or cracked skin in the area of the chord wise weights. Leading edge for erosion, nicks, dents, or scratches. Bonded joints of blade skin mating area for suspected voids or cracks. Feathering bearings for looseness. |  |  |

| - POST INSPECTION |  |  |
| --- | --- | --- |
| - Inspect for operation and leaks. |  |  |
| - Replace or close all applicable trim panels, covers, access and inspection doors/covers as necessary. |  |  |
| - Perform Functional Check Flight if required per Bell 206L Series Maintenance Manual or RFM as appropriate. |  |  |
| - Make an entry in the appropriate logbook (refer to the following page for wording of approval for return to service). |  |  |

Upon satisfactory completion of each inspection item, the mechanic shall initial the pertinent space as provided.  All mechanics shall sign and initial, in the space provided, at the end of the form.

At the completion of each inspection form, an A&P mechanic shall check the entire form for completion and then enter his/her signature, certificate type and number, and date in the space provided at the end of the form.  The same individual is also responsible for making the logbook entry shown at the conclusion of this inspection.

I certify that I have inspected all the items initialed by me in accordance with Maritime Helicopters’ Bell 206L Series Approved Aircraft Inspection Program and found them to be in an airworthy condition except as may be noted in the Aircraft Logbook.

| INITIALS | SIGNATURE | CERTIFICATE TYPE & NUMBER |
| --- | --- | --- |

I have reviewed this inspection form to verify that all items have been completed and have made an entry in the Aircraft Log Book.

I certify that this aircraft has been inspected in accordance with Maritime Helicopters’ Bell 206L Series AAIP – A0056 – 50hr Inspection and was determined to be in an airworthy condition.

| Registration No.: | N | End |  | Aircraft Ending Total Time: |  |
| --- | --- | --- | --- | --- | --- |
| Aircraft S/N: |  | Date: |  | Aircraft Ending Total Cycles: |  |

| Signature: |  |  | Certificate Type & No: |  |
| --- | --- | --- | --- | --- |