Federal Aviation Administration

MMEL Policy Letter 105, Revision 2

Date:	12/04/2017

To:	Manager, Aircraft Evaluation Division (AFS-100)
All Aircraft Evaluation Group Managers

From:	Manager, Air Transportation Division (AFS-200)

Reply to Attn of:	Manager, New Program Implementation Branch (AFS-240)

SUBJECT:	Automatic Dependent Surveillance-Broadcast (ADS-B) System

MMEL CODE:	34 (NAVIGATION)

REFERENCE:	PL-105, Revision 1, dated January 20, 2009
PL-105, Original, dated October 11, 2000
Title 14 of the Code of Federal Regulations (14 CFR) part 91, §§ 91.225, 91.227, Advisory Circular (AC) 20-165, AC 20-172

PURPOSE:

The purpose of this Policy Letter is to provide standardized Master Minimum Equipment List (MMEL) requirements for Automatic Dependent Surveillance-Broadcast (ADS-B) systems.

DISCUSSION:

Revision 2: Removes the Global Change designation and replaces the previous discussion that addressed a specific ADS-B system installation with information applicable to all ADS-B installations.

The following discussion uses excerpts from the Federal Registry Docket No. FAA-2007-29305; Amdt. No. 91-314 (14 CFR Part 91 ADS-B Out final rule), FAA AC 20-165 (guidance for ADS-B Out systems) and FAA AC 20-172 (guidance information for ADS-B In systems).

As part of NextGen development, the FAA has determined that it is essential to move from ground-based surveillance and navigation to more dynamic and accurate airborne-based systems and procedures if the agency is to enhance capacity, reduce delay, and improve environmental performance. Automatic Dependent Surveillance-Broadcast (ADS-B) equipment is an advanced surveillance technology that combines an aircraft’s positioning source, aircraft avionics, and a ground infrastructure to create an accurate surveillance interface between aircraft and air traffic control (ATC). ADS-B is a key component of NextGen that will move ATC from a radar-based system to a satellite-derived aircraft location system. ADS-B is a performance-based surveillance technology that is more precise than radar. ADS-B is expected to provide air traffic controllers and pilots with more accurate information to help keep aircraft safely separated in the sky and on runways. The technology combines a positioning capability, aircraft avionics, and ground infrastructure to enable more accurate transmission of information from aircraft to ATC.

ADS-B consists of two different functions: ADS-B Out and ADS-B In. ADS-B Out periodically broadcasts information about each aircraft, such as identification, current position, altitude, and velocity, through an onboard transmitter. ADS-B Out provides air traffic controllers with real-time position information that is, in most cases, more accurate than the information available with current radar-based systems. With more accurate information, ATC will be able to position and separate aircraft with improved precision and timing.

ADS-B In refers to an appropriately equipped aircraft’s ability to receive and display another aircraft’s ADS-B Out information, as well as the ADS-B In services provided by ground systems, including Automatic Dependent Surveillance-Rebroadcast (ADS-R), Traffic Information Service-Broadcast (TIS-B), and, if so equipped, Flight Information Service-Broadcast (FIS-B). When displayed in the cockpit, this information greatly improves the pilot’s situational awareness.

ADS-B requires a broadcast link for aircraft surveillance and to support ADS-B In applications. Operators have two options for equipage under this rule: the 1090 extended squitter (1090ES) broadcast link with performance requirements specified in TSO-C166b or the Universal Access Transceiver (UAT) broadcast link that operates on 978 MHz and has performance requirements specified in TSO-C154c.

The 1090 MHz ES broadcast link is the internationally agreed upon link for ADS-B and is intended to support ADS-B Out and ADS-B In in applications used by air carriers and other high-performance aircraft. The 1090 MHz ES broadcast link does not support FIS-B (weather and related flight information) because the bandwidth limitations of this link cannot transmit the large message structures required by FIS-B. The UAT link supports both ADS-B Out and ADS-B In, as well as FIS-B.

Beginning January 1, 2020, all aircraft operating in airspace defined in § 91.225 will be required to have ADS-B Out which meets the requirements of §§ 91.225 and 91.227. ADS-B In will not be required.

POLICY:

The following MMEL proviso and repair interval is adopted to provide standardization among all MMELs:

34. NAVIGATION

| Sequence No. | Item | 1 | 2 | 3 | 4 | Change Bar |
| --- | --- | --- | --- | --- | --- | --- |
| 34-XX *** | Automatic Dependent Surveillance-Broadcast (ADS-B) System | C | - | 0 | (O) May be inoperative provided: - Alternate procedures are established and used, and - It is not required by 14 CFR. NOTE: Any ADS-B function that operates normally may be used. |  |
|  |  | D | - | 0 | May be inoperative provided: - Enroute operations do not require its use, and - It is not required by 14 CFR. NOTE: Any ADS-B function that operates normally may be used. |  |
|  |  | C | - | 1 | One must be operative as required by 14 CFR. NOTE: Any ADS-B function that operates normally may be used. |  |
| *** | ADS-B Out Extended Squitter Transmissions | C | - | 0 | (O) May be inoperative provided: - Alternate procedures are established and used, - Authorization is obtained from ATC facilities having jurisdiction over planned route of flight, and - It is not required by 14 CFR. NOTE: Any ADS-B function that operates normally may be used. |  |
|  |  | C | - | 1 | One must be operative as required by 14 CFR. NOTE: Any ADS-B function that operates normally may be used. |  |
| *** | ADS-B Out UAT Transmissions | C | - | 0 | (O) May be inoperative provided: - Enroute operations do not require its use, - Authorization is obtained from ATC facilities having jurisdiction over planned route of flight, and - It is not required by 14 CFR. NOTE: Any ADS-B Out function that operates normally may be used. |  |
|  |  | C | - | 1 | One must be operative as required by 14 CFR. NOTE: Any ADS-B function that operates normally may be used |  |
| *** | ADS-B In Transmissions | C | - | 0 | (O) May be inoperative provided alternate procedures are established and used. NOTE: Any ADS-B In function that operates normally may be used. |  |
|  |  | D | - | 0 | May be inoperative provided operations do not require its use. NOTE: Any ADS-B function that operates normally may be used. |  |

Each Flight Operations Evaluation Board (FOEB) Chair should review the specific aircraft configuration(s) and apply this policy to affected MMELs through the normal FOEB process.

ORIGINAL SIGNED by

/s/ Stephen C. Moates for

Manager, Air Transportation Division