REPAIR STATION

FORMS MANUAL

For

FAA Approved Repair Station No ENRR619D

FAA Approved Satellite Repair Station No EN2D619D

DBA

Maritime Helicopters, Inc.

3520 FAA Road

Homer, Alaska 99603

(907) 235-7771

Revision 5

07/01/21

Published by:

Maritime Helicopters, Inc.

3520 FAA Road

Homer, AK  99603

(907) 235-7771

Copyright © 2013 Maritime Helicopters, Inc.

All Rights Reserved. No part of the contents of this document may be reproduced or transmitted in any form by any means without the written permission of Maritime Helicopters, Inc

Record of Revisions

| Rev No. | Rev Date | Change Summary | Posted |
| --- | --- | --- | --- |
| Reissue | 11/01/2013 | All Pages | SDS |
| Rev 1 | 01/10/2015 | ii, iii, iv, 3-1, 4-38, 4-39, 4-40; 411/413 Forms | SDS |
| Rev 2 | 07/20/2015 | Cover, ii, iii, iv, 3-1, 4-41; Added Return to Service Roster | SDS |
| Rev 3 | 08/15/2016 | Cover, ii, iii, iv, 3-1, 4-7-11,4-30, 4-31 4-50-52 Added forms | SDS |
| Rev 4 | 04/01/2019 | Cover, ii, iii, iv, v, 3-1, 3-2,4-53 thru 5-48; Added forms | SDS |
| Rev 5 | 07/01/2021 | Cover, iii – 46; Added/deleted some forms. Rearranged order | SDS |

List of Effective Pages

| PAGE | STATUS | DATE |  | PAGE | STATUS | DATE |  | PAGE | STATUS | DATE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i | Rev 5 | 07/01/21 |  | 25 | Rev 5 | 07/01/21 |  |  |  |  |
| ii | Reissue | 11/01/13 |  | 26 | Rev 5 | 07/01/21 |  |  |  |  |
| iii | Rev 5 | 07/01/21 |  | 27 | Rev 5 | 07/01/21 |  |  |  |  |
| iv | Rev 5 | 07/01/21 |  | 28 | Rev 5 | 07/01/21 |  |  |  |  |
| v | Rev 5 | 07/01/21 |  | 29 | Rev 5 | 07/01/21 |  |  |  |  |
| 1 | Rev 5 | 07/01/21 |  | 30 | Rev 5 | 07/01/21 |  |  |  |  |
| 2 | Rev 5 | 07/01/21 |  | 31 | Rev 5 | 07/01/21 |  |  |  |  |
| 3 | Rev 5 | 07/01/21 |  | 32 | Rev 5 | 07/01/21 |  |  |  |  |
| 4 | Rev 5 | 07/01/21 |  | 33 | Rev 5 | 07/01/21 |  |  |  |  |
| 5 | Rev 5 | 07/01/21 |  | 34 | Rev 5 | 07/01/21 |  |  |  |  |
| 6 | Rev 5 | 07/01/21 |  | 35 | Rev 5 | 07/01/21 |  |  |  |  |
| 7 | Rev 5 | 07/01/21 |  | 36 | Rev 5 | 07/01/21 |  |  |  |  |
| 8 | Rev 5 | 07/01/21 |  | 37 | Rev 5 | 07/01/21 |  |  |  |  |
| 9 | Rev 5 | 07/01/21 |  | 38 | Rev 5 | 07/01/21 |  |  |  |  |
| 10 | Rev 5 | 07/01/21 |  | 39 | Rev 5 | 07/01/21 |  |  |  |  |
| 11 | Rev 5 | 07/01/21 |  | 40 | Rev 5 | 07/01/21 |  |  |  |  |
| 12 | Rev 5 | 07/01/21 |  | 41 | Rev 5 | 07/01/21 |  |  |  |  |
| 13 | Rev 5 | 07/01/21 |  | 42 | Rev 5 | 07/01/21 |  |  |  |  |
| 14 | Rev 5 | 07/01/21 |  | 43 | Rev 5 | 07/01/21 |  |  |  |  |
| 15 | Rev 5 | 07/01/21 |  | 44 | Rev 5 | 07/01/21 |  |  |  |  |
| 16 | Rev 3 | 08/15/16 |  | 45 | Rev 5 | 07/01/21 |  |  |  |  |
| 17 | Rev 3 | 08/15/16 |  | 46 | Rev 5 | 07/01/21 |  |  |  |  |
| 18 | Rev 5 | 07/01/21 |  |  |  |  |  |  |  |  |
| 19 | Rev 5 | 07/01/21 |  |  |  |  |  |  |  |  |
| 20 | Rev 5 | 07/01/21 |  |  |  |  |  |  |  |  |
| 21 | Rev 5 | 07/01/21 |  |  |  |  |  |  |  |  |
| 22 | Rev 5 | 07/01/21 |  |  |  |  |  |  |  |  |
| 23 | Rev 5 | 07/01/21 |  |  |  |  |  |  |  |  |
| 24 | Rev 5 | 07/01/21 |  |  |  |  |  |  |  |  |

_____________________________________________	      _____________________________________________

FAA PRINCIPAL MAINTENANCE INSPECTOR		              FAA PRINCIPAL AVIONICS INSPECTOR

Table of Contents

Record of Revisions	iii

List of Effective Pages	iv

Table of Contents	v

1.	Introduction	1

1.1	Manual Revisions	1

1.2	Manual Revisions and Control	1

1.3	Web Based Manual	1

Forms Listing	2

REPAIR STATION FORMS	3

1.	Authorized Release Certificate FAA Form 8130-3 – MHI 001	3

2.	Work Order Cover Sheet – FORM MHI 002	6

3.	Vendor Audit – FORM MHI 003	8

4.	Repairable Parts Tag – Form MHI 004	13

5.	Part 145 Work Sheet – FORM MHI 005	15

6.	Serviceable Parts Tag – FORM MHI 006	18

7.	Rejected Part Tag – Form 007	20

8.	Identification Tag - FORM MHI 008	22

9.	Receiving Inspection Log – FORM MHI 009	24

10.	Non-Destructive Testing Parts Tag – FORM MHI 010	26

11.	NDT Inventory Log – FORM MHI 011	28

12.	Shelf Life Audit – FORM MHI 012	30

13.	Inventory Part Receiving Label – FORM MHI 013 (Fairbanks Only)	31

14.	Inventory Part Receiving Label – FORM MHI 014 (Homer Only)	32

15.	Summary of Employment – FORM MHI 015	33

16.	Calibration Sticker (sample) – FORM MHI 016	35

17.	Capabilities Self Evaluation Checklist – FORM 017	36

18.	 Tool Calibration Record – MHI Form 018	38

19.	Inventory Sign Out Tag – Form MHI 019	39

20.	Required Logbook Entries Tag – Form MHI 020	40

21.	Mechanic Signature/Initial Roster - Form MHI 021	41

22.	Altimeter 24 Month Checklist Form MHI 022	42

23.	Transponder 24 Month Checklist Form MHI 023	44

24.	Repair Station Inspection and Return to Service Roster - Form MHI 024	45

25.	Repair Station Management and Supervisory Roster - Form MHI 025	46

## Introduction

This Repair Station Forms Manual (RSFM) has been prepared in accordance with the current regulations contained in Title 14 CFR §145, and is a prerequisite for operation under the privileges of the Repair Station certificate. The validity of the Repair Station’s certificate is directly dependent upon compliance with the procedures and requirements within this manual, as well as, the associated Repair Station and Quality Control Manuals.

MHI Repair Stations will not maintain or alter any article for which it is not rated, and will not maintain or alter any article for which it is rated if it requires technical data, equipment, materials, facilities or trained personnel that are not available.

A current copy of this RSFM, including the associated Repair Station and Quality Control Manuals, shall be accessible to applicable Repair Station personnel. A current copy of the required Repair Station Forms Manual and any revisions made will also be provided to the FAA Certificate Holding District Office (CHDO).

### Manual Revisions

The Accountable Manager oversees the creation and revision of MHI controlled documents. Department Managers and Supervisors may submit suggestions and corrections for incorporation into this manual by contacting the Chief Inspector. Changes made to this manual will be summarized in the change summary table and indicated throughout the manual by change bars. A vertical bar (change bar) in the right margin indicates a change, addition, or deletion in the adjacent text for the current revision of that page only.  The change bar is dropped at the next revision of that page.

### Manual Revisions and Control

The Chief Inspector is responsible for the content of this manual and any revisions. This manual requires FAA acceptance prior to release. Revisions to this manual must be routed for MHI internal and FAA acceptance prior to distribution. This manual cannot be published or distributed to end users prior to formal submission to and acceptance by the FAA.

### Web Based Manual

Web based company manuals maintained by the Part 145 Accountable Manager will be placed on the Maritime Helicopters company maintenance Portal and available to all Part 145 maintenance personnel. The Part 145 Accountable Manager is responsible for assuring all web-based company manuals are current.  Any downloaded or printed material from the company maintenance Portal is considered an uncontrolled copy.

Maintenance Personnel are to use the Maritime Maintenance Web Portal for all company manuals unless internet access is not available.  In the advent of no internet access to manuals, a current company paper format manual shall be used.

Forms Listing

| Form # | Description |
| --- | --- |
| MHI 001 | FAA Form 8130-3 |
| MHI 002 | Work Order Cover Sheet |
| MHI 003 | Vendor Audit |
| MHI 004 | Repairable Parts Tag |
| MHI 005 | Part 145 Work Sheet |
| MHI 006 | Serviceable Parts Tag |
| MHI 007 | Rejected Parts Tag |
| MHI 008 | Identification Tag |
| MHI 009 | Receiving Inspection Log |
| MHI 010 | NDT Testing Parts Tag |
| MHI 011 | NDT Inventory Log |
| MHI 012 | Shelf-Life Audit |
| MHI 013 | Inventory Part Receiving Label (Fairbanks Only) |
| MHI 014 | Inventory Part Receiving Label (Homer Only) |
| MHI 015 | Summary of Employment |
| MHI 016 | Calibration Sticker |
| MHI 017 | Capabilities Self-Evaluation Checklist |
| MHI 018 | Tool Calibration Record |
| MHI 019 | Inventory Sign Out Tag |
| MHI 020 | Required Logbook Entries |
| MHI 021 | Mechanic Signature/Initial Roster |
| MHI 022 | Altimeter 24 Month Checklist |
| MHI 023 | Transponder 24 Month Checklist |
| MHI 024 | Inspection and Return to Service Roster |
| MHI 025 | Management and Supervisor Roster |

REPAIR STATION FORMS

## Authorized Release Certificate FAA Form 8130-3 – MHI 001

APPLICABILITY: Only Maritimes Helicopters Fairbanks Overhaul Shop is required to issue an FAA Form 8130-3 in lieu of yellow serviceability parts tag (FORM MHI 006) for work performed under a FORM MHI 002 work order.

Electronic copies of each completed FAA Form 8130-3 shall be submitted to the Part 145 Accountable Manager or his designee for review.

Instructions for completion of FAA Form 8130-3:

Block #

1. Approving National Aviation Authority/Country - FAA/United States. Preprinted

2. Authorized Release Certificate, FAA Form 8130-3, Airworthiness Approval Tag - Preprinted

3. Form Tracking Number - Enter the unique number established by the numbering system.

4. Organization Name and Address - Enter the full name and physical address (no post office box numbers) of the organization or facility for which the form is being issued and Repair Station Number (EN2D619D).

5. Work Order/Contract/Invoice Number - To facilitate customer traceability of the product, part, or appliance, enter the work order number, contract number, invoice number, or similar reference number.

6. Item - When Form 8130-3 is issued, a single item number or multiple item numbers (for example, same item with different serial numbers) may be used for the same part number. Multiple items must be numbered in sequence, although not necessarily beginning with the number one (for example, 0040, 0050, 0062, 0063). If a separate listing is used, enter “List Attached”

7. Description - Enter the name or description of the product, part, or appliance. Preference should be given to the term used in the instructions for continued airworthiness or maintenance data (for example, illustrated parts catalog, aircraft maintenance manual, or service bulletin (SB)).

8. Part Number - Enter each part number of the product, part, or appliance. If article is a subassembly and does not have a part number, enter the next higher assembly part number followed by the words “Subassembly”.

9. Quantity - Enter the quantity of each product, part, or appliance shipped.

10. Serial/Batch Number - If the product, part, or appliance is required by 14 CFR part 45, Identification and Registration Marking, to be identified with a serial number, enter it here. Additionally, any other serial number not required by regulation also may be entered. If no serial number is entered in this block, enter “N/A.”

11. STATUS/WORK – enter one of the following statements accordingly: (1) TESTED, (2) INSPECTED, (3) REPAIRED, (4) MODIFIED, (5) OVERHAULED, or (6) SEE BLOCK 12.

12. Remarks - State any information in this block, either directly or by reference, to supporting documentation necessary for the user or installer to determine the airworthiness of the product, part, or appliance. If necessary, a separate sheet may be used and referenced from the main Form 8130-3. Each statement must clearly identify which product, part, or appliance in Block 6 it relates to. If there is no statement, state “none.” The purpose of the form (for example, airworthiness approval.

13. Blocks 13a through 13e. Shade, darken, or otherwise mark to preclude inadvertent or   unauthorized use.

14a. Approval for Return to Service - Mark the box 14 CFR 43.9 Return to Service.

14b. Authorized Signature - This space will be completed with the signature of the authorized person. Only persons specifically authorized are permitted to sign this block. The approval signature must be applied at the time and place of issuance and manually applied,

14c. Approval/Certificate No. Enter the air agency, or air carrier certificate number (for example, EN2D619D).

14d. Name - Enter the typed or printed name of the authorized representative whose signature appears in Block 20.

14e. Date (m d y) - Enter the date on which Block 14 is completed, or in the case of electronically generated forms, the date the conformity determination is made and the form is authorized to be issued. The date must be in the following format: first three letters of the month, two-digit day, and four-digit year, for example, Feb 03 2008. This does not need to be the same as the printing or shipping date, which may occur later.

NOTE: Lost Form 8130-3 Issued for an Approval for Return To Service. If a copy of a Form 8130-3 is requested, a file copy of the original form may be provided by an authorized person, if available.

Reissuance of Form 8130-3 Because of Typographical Errors on the Original - The original issuer may reissue Form 8130-3 if there are typographical errors on the original.

a. The recipient must provide a written statement and a copy of the incorrect Form 8130-3 to the original issuer highlighting the errors.

b. Once these actions are taken, the copy of the original form should be reviewed to determine validity of the errors. If the errors are valid, a corrected form may be issued and the words “THIS FORM 8130-3 REPLACES FORM 8130-3 WITH FORM TRACKING NUMBER [insert number], DATED [enter original issuance date]” must be typed in Block 13. The replacement form must have an original signature and the date the signature was applied. The erroneous form must be marked as such.

NOTE: Both forms should be retained according to the retention period associated with the original form.

## Work Order Cover Sheet – FORM MHI 002

Form 002										                                  07/01/2013

Instructions: Work Order Cover Sheet

This form shall be utilized to document the aircraft information and work to be performed. Complete the fields as follows:

| W.O. # | Enter a work order number for tracking |
| --- | --- |
| Start | Date the work order was opened |
| Customer Name | Customer’s name |
| Complete | Date the work order was closed |
| Address | Street address of company |
| Phone | Contact phone number |
| Labor | Total mechanic hours |
| Rate | Agreed upon shop rate |
| City & State | City & state of customer address |
| Make & Model | For example, 'Bell 407’ |
| Serial # | For an aircraft, enter serial aircraft serial number. For an article, refer to the serviceability tag and enter the S/N of the aircraft from which the article was removed or the article’s S/N. |
| N# | For the aircraft, enter serial aircraft serial number. For an article, refer to the serviceability tag. Enter N# of the aircraft from which the article was removed. |
| Total Time | The total time for an aircraft. For an article, the aircraft’s total flight time at time of removal. |
| RIN | Fill in cycles, landings, or flights for articles or assembly (as applicable) |
| Engine TT | Total time of aircraft engine |
| Cycles / Starts | Engine/prop can be found on serviceability tag, status report, or logbook. |
| Labor | Total labor cost |
| Parts | Total cost of parts |
| Outside Repairs | Total cost of outside repairs |
| Gas | Total cost of fuel |
| Oil | Total cost of oil |
| Shipping | Total cost of shipping |
| Tax | Amount of tax costs applied |
| Grand Total | Total costs |
| Authorization | Signature and date of person(s) authorizing the aircraft work |

COMPONENTS

| Name | Common abbreviations are acceptable (i.e., M/R, T/R, G/B, Assy, etc.) |
| --- | --- |
| Part # | Part number of the article from serviceability tag. If there are multiple articles, write "See below" and list the part numbers with each specific article in the Job Description. |
| Serial # | Serial number of the article from serviceability tag. If there are multiple articles, write "See below", list the serial numbers with each specific article in the job description. |
| Total Time/TSO | Applicable total time (since new) of the article or time since overhaul. If there are multiple articles, write “See below”, list the times with each specific article in the job description. |

## Vendor Audit – FORM MHI 003

Maritime Helicopters

Repair/Overhaul Vendor Audit Surveillance Report

| Vendor Name: | Phone: Fax: |
| --- | --- |
| Vendor Contact Name: |  |
| Address: |  |

| Key Company Officials: |  |
| --- | --- |
| Name | Title |

| Individual Responsible for Quality Control Program: |  |  |
| --- | --- | --- |
| Name | Title | Phone |

| Type of Business: | Airline |  | Manufacturer |  | FAA Repair Station |
| --- | --- | --- | --- | --- | --- |
| Distributor/Supplier |  | Other |  |  | Repair Station # |
| Years in Business: |  |  |  | Number of Employees: |  |

| CAA or FAA Principal Maintenance Inspector: |  |
| --- | --- |
| Name | Phone |

| Major Customers: |  |
| --- | --- |
| Company Name | Service Provided |

| Personnel Numbers: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Production | Quality | Engineering |  |  | Certified Repairmen |  |
|  |  |  | Yes | No |  | N/A |
| 1) Certification |  |  |  |  |  |  |
| a) Do you have a FAR Part 145 Repair Station Certificate? |  |  |  |  |  |  |
| i) Please attach a copy of the certificate and operations specifications to this questionnaire. |  |  |  |  |  |  |
| b) Do you have an approved anti-drug program? |  |  |  |  |  |  |
| i) Please attach a copy of the FAA acceptance/exemption letter. |  |  |  |  |  |  |
| c) Do you have an approved alcohol testing program? |  |  |  |  |  |  |
| i) Please attach a copy of the FAA acceptance/exemption letter. |  |  |  |  |  |  |
| 2) Quality Control Program |  |  |  |  |  |  |
| a) Do you have an established quality control program? |  |  |  |  |  |  |
| b) Is it described in detail in a quality control manual or other appropriate document? |  |  |  |  |  |  |
| c) Please describe the functional separation of Quality Control and Production. |  |  |  |  |  |  |
| d) Are all inspections and tests documented? |  |  |  |  |  |  |
| e) Are all incoming parts and materials subject to a documented receiving inspection? |  |  |  |  |  |  |
| 3) Technical Data |  |  |  |  |  |  |
| a) Do you have the required technical data on-hand to perform the work we will be contracting/are sending to you? |  |  |  |  |  |  |
| b) Do you have an established system to ensure technical data remains current? |  |  |  |  |  |  |
| c) Is uncontrolled technical/reference data marked as such and segregated from controlled data? |  |  |  |  |  |  |
| 4) Tooling and Test Equipment |  |  |  |  |  |  |
| a) Do you have adequate tooling and test equipment necessary to perform the work we will be contracting/are sending to you? |  |  |  |  |  |  |
| b) Do you have an established system to ensure company and personal tools and test equipment are maintained serviceable/calibrated? |  |  |  |  |  |  |

|  | Yes | No | N/A |
| --- | --- | --- | --- |
| 5) Training |  |  |  |
| a) Do you have a formal training program for all supervisors, inspectors, and technicians? |  |  |  |
| b) Do you document all training? |  |  |  |
| 6) Facilities |  |  |  |
| a) Do you have adequate facilities to perform the work we will be contracting/are sending to you? |  |  |  |
| b) Do you have a system that ensures parts, materials, and customer property is protected from damage, theft, and contamination? |  |  |  |
| c) Are non-aircraft related activities segregated from aircraft related activities? |  |  |  |
| 7) Work Processing |  |  |  |
| a) Do you have a system to properly identify customer parts throughout all maintenance actions and while in storage? |  |  |  |
| b) Are serviceable and unserviceable units kept segregated? |  |  |  |
| c) Are scrap parts tagged and removed from the work area? |  |  |  |
| 8) Stores |  |  |  |
| a) Do you have a system to ensure all parts and materials are environmentally protected? |  |  |  |
| b) Do you maintain traceability certification for all parts and materials? |  |  |  |
| c) Are aircraft parts segregated from non-aircraft parts? |  |  |  |
| d) Do you have a quarantine area for rejected parts and materials awaiting disposition? |  |  |  |
| e) Are life limited scrap parts returned to the customer or mutilated to prevent return to service? |  |  |  |
| f) Do you have a system that monitors shelf-life limited parts and supplies? |  |  |  |

| Vendor Comments: |  |
| --- | --- |
| The data herein is applicable to the execution of contracts and the filling of orders from Maritime Helicopters, Inc. It is agreed that Maritime Helicopters, Inc. will be notified of any relocation or transfer of operations, or any change in quality control organization or procedures which affects compliance of relevant supplies and procedures to Maritime Helicopters, Inc. requirements. It is recognized that failure to furnish a description of such changes for Maritime Helicopters, Inc. review, or willful misrepresentation of any fact specified herein constitutes a material breach of contract which may result in disapproval as a Maritime Helicopters, Inc. vendor and void any existing contract purchase order. |  |
| Signature of Company Official | Position Title |
| Name | Phone |

This Section for Maritime Helicopters, Inc. Use Only

Work Contracted/Intended:

| Type Audit: | Initial | Recurrent | Follow-Up | Special |
| --- | --- | --- | --- | --- |

| Corrective Action Required |  | Date Requested | Date Due: | Date Received: |
| --- | --- | --- | --- | --- |
| Yes | No |  |  |  |

| Approved for Use: | Yes | No | By: | Date: |
| --- | --- | --- | --- | --- |
| Approved Vendor List Updated: |  |  |  |  |

Next Scheduled Audit Date:

Comments:

| Records Accounting: | Completed Checklist |  | Finding/corrective Action Form(s) |  |  |
| --- | --- | --- | --- | --- | --- |
| Repair Station Certificate |  | Operations Specifications |  | Drug Letter | Alcohol Letter |

Instructions: Vendor Audit Form

This form shall be utilized to document Maritime repair and overhaul vendors in performing a self-audit.

## Repairable Parts Tag – Form MHI 004

Instructions: Repairable Parts Tag

This tag shall be utilized to record the information of a repairable article. It is initiated by a mechanic. It will remain with the article until the article is rejected or made serviceable.

| Part Name | Part name. |
| --- | --- |
| Date | Date of removal. |
| Part No. | Part number. |
| Serial No. | Part serial number. |
| Aircraft N# | Aircraft registration number. |
| Position | If multiple locations i.e., Left hand, Upper |
| TSO | Time Since Overhaul, If applicable. |
| Total Time | Total time of part. |
| Reason Removed | Why removed; Unserviceable, overhaul, etc. |
| Mechanic | Signature of mechanic that removed the part |
| Inspector | Name of approving inspector, if required. |

## Part 145 Work Sheet – FORM MHI 005

Sample: Maintenance Discrepancy and Work Record

Instructions: Maintenance Discrepancy and Work Record

This form shall be utilized to document discrepancies and corrective actions. Upon completion, this form will become part of the work order package.

| Registration No. | Enter aircraft registration number. |  |
| --- | --- | --- |
| Aircraft Model | For example, 'Bell 407’ |  |
| Location | Where maintenance is being performed |  |
| Work Order No | Found at the top of the MHI form 002 |  |
| Acft TT | Aircraft total current airframe time |  |
| Page No. | Page number if multiple pages are used. |  |
| Item # | Item # for sequential itemization of different tasks to be performed (i.e. for multiple steps: #1, #2, #3, etc.). |  |
| Date discovered | The person that entered the discrepancy must input date of the entry. |
| Pre Insp/In Progress | Note type of inspection. In Progress if item is being inspected as part of a larger assembly |  |
| S/N Off | Enter serial number (If serialized) of component removed |  |
| S/N On | Enter serial number (If serialized) of component installed |  |
| Discrepancy | Use this space to describe the removal or discrepancy. |  |
| Corrective Action | State the installation/corrective action within the section provided. References must satisfy requirements per CFR 14 § 43.13. i.e. MM, dwg, ICA. |  |
| Discovered by | Mechanic who found the discrepancy will enter their initials |  |
| Corrective by signature | The mechanic that entered the corrective action will enter their initials. |
| Certificate | Mechanics certificate number (may be Repair Station number) |
| Final Insp | QA inspector will enter their initials if all work has been completed satisfactorily. |
| Date | Enter date of completion of item |  |

ADDITIONAL INFO

| Name | Common abbreviations are acceptable (i.e., M/R, T/R, G/B, Assy, etc.) |
| --- | --- |
| Part # | Part number of the article from serviceability tag. If there are multiple articles, write "See below" and list the part numbers with each specific article in the Job Description. |
| Serial # | Serial number of the article from serviceability tag. If there are multiple articles, write "See below", list the serial numbers with each specific article in the job description. |
| Total Time/TSO | Applicable total time (since new) of the article or time since overhaul. If there are multiple articles, write “See below”, list the times with each specific article in the job description. |

| Item No. | For sequential itemization of different jobs (i.e. multiple articles: #1, #2, #3, etc.). |
| --- | --- |
| Description | Fill in the job description from the discrepancy on the Component record tag or an abbreviated version from the TSD for an aircraft. For more than one article, include the P/N, S/N, Name and applicable TSN/TSO times. |

INSPECTIONS

| Preliminary | A preliminary inspection MUST BE PERFORMED on any aircraft or article prior to maintenance being performed. Initials in this box stating an inspection was performed. If a preliminary discrepancy(s) was/were found, write up discrepancies(s) were found in box. |
| --- | --- |
| Hidden Damage | The QA inspector shall write up any hidden damage if found. If no hidden damage is found, write "N/A" for any hidden. |
| Continuity | QA Inspector initials. “See Attached” - continuity forms were used; “No” if continuity forms were not used. |
| Maintenance Final | Shall be initialed by the assigned Lead or Supervisor upon completion of work performed. |
| QA Final | The QA inspector shall acknowledge that the work record has had a final review prior to being closed and write in that statement. |

## Serviceable Parts Tag – FORM MHI 006

FRONT

BACK

Instructions: Serviceable Parts Tag

This tag shall be utilized to record the information of a serviceable article. It is initiated by a mechanic. It will remain with the article is installed.

| Nomenclature | Part name. |
| --- | --- |
| Date | Date of serviceability. |
| Part No. | Part number. |
| Serial No. | Part serial number. |
| TSO | Time Since Overhaul if applicable. |
| Total Time | Total time of part. Also enter a RIN count if applicable |
| Mechanic | Signature required of the mechanic removed the part. |
| Inspector | Signature required of the QA inspector or the mechanic condemning the part. |
| INSTALLATION INFO |  |
| Aircraft N# | Aircraft registration number. |
| Date | Date of installation. |
| Acft TT | Current aircraft airframe total time |
| Position | I.e. L/H, Upper, etc. |
| Mechanic | Installing mechanic name |
| Inspector | Installation Inspector |
| MAINTENANCE RELEASE |  |
| W.O # | Work Order number (if applicable) |
| Date | Date of Release. |
| Signed | Signature of releasing inspector |
| OVERHAUL AGENCY |  |
|  | Circle status of part. i.e., new, overhauled, repaired, etc. |
| MHI PO# | Enter MHI Purchase Order number |

## Rejected Part Tag – Form 007

Instructions: Rejected Part Tag

This tag shall be utilized to record the information of a rejected article. It is initiated by a mechanic. It will remain with the article until the article is destroyed or mutilated.

| Date | Date of rejection. |
| --- | --- |
| Part Name | Part name. |
| Part No. | Part number. |
| Serial No. | Part serial number. |
| Total Time | Total time of part. |
| TSO | If applicable. |
| RIN | If applicable. |
| CYC | If applicable. |

| Aircraft N# | Aircraft registration number. |
| --- | --- |
| TT | If applicable. |
| Reason | The reason for rejection must include a detailed explanation of why the part is not serviceable. |
| Methods Destruction | Record methods used to render the part un-usable. Prior to destruction, ensure parts are not to be returned to the owner. |
| Work Order No | Work order number. |
| Item No | Discrepancy item in the work order originally rejecting the part. |
| Inspector/Mech | Signature required of the QA inspector or the mechanic condemning the part. |
| Cert Type & No | Enter the applicable repair station number if signing for an MHI operated repair station. |

## Identification Tag - FORM MHI 008

Instructions: White Identification Tags

This tag shall be utilized to identify articles or shelving/storage units as required. It is initiated by a mechanic. This tag may be discarded when the component(s) is reinstalled on the same aircraft or identification tag is replaced by a component record card (form MHI 006).

| Work Order No. | Enter work order No. from applicable form MHI 002. |
| --- | --- |
| Item No. | Enter discrepancy item number from form MHI 005. |
| P/N Description | Enter part number of article and describe. |
| & S/N | Enter article serial number. |
| Removed from N#/S/N | Enter next higher assembly information. |

Side One: ITEMS REMOVED FROM AIRCRAFT / ASSEMBLY

With the information on this side completed, the tag shall be attached to indicate a serviceable article’s removal from an aircraft "for maintenance convenience only”. This shall be attached to the individual item. Should the individual articles labeled with an "Identification Tag,” form MHI 008, require any maintenance, enter a work order discrepancy card entry on form MHI 005. If the removed article is not to be reinstalled on the original aircraft, it requires the article to be tagged with a component record tag (form MHI 006).

NOTE: No item removed and tagged with a white Identification Tag, form MHI 008 will be reinstalled unless the item is deemed “serviceable” by an appropriately rated mechanic.

Side Two: ALL ITEMS PLACED ON OR IN THIS STORAGE UNIT SIDE

With the information on this side completed, the tag shall be utilized to identify shelves, racks and containers.

In the case of cabin interiors (seats, upholstery, etc.), empennage or fuselage cowls, inspection/access panels, hardware, and other similar items, such parts must be serviceable and will be segregated on shelves, racks or in containers labeled as appropriate with one identification tag.

## Receiving Inspection Log – FORM MHI 009

Instructions: Receiving Inspection Log

This form provides a record of articles received into the repair station. It is initiated by an incoming/receiving Inspector. The form is to be archived in the incoming and receiving area.

| Date Received | Date item entered into the log. |
| --- | --- |
| PO# | PO number as stated on invoice. |
| Part # | Number of part. |
| Serial # | Serial number affixed to part. |
| QTY | Total number of parts received as stated on invoice. |
| Description | As stated on invoice. |
| Shelf Life | Indicate YES or NO whether shelf life is limited. |
| Maint Release | Indicate YES or NO whether maintenance release was supplied with unit. |
| Insp Initial | Initials of incoming and receiving inspector completing the inspection. |

## Non-Destructive Testing Parts Tag – FORM MHI 010

Instructions: Non-Destructive Testing Parts Tag

This tag is used to identify articles leaving an MHI repair station for non-destructive testing. It is initiated by a mechanic.  After work is performed by the vendor this form is no longer required for the work order package.

| WO # | Assigned work Order number from form MHI 002. |
| --- | --- |
| N# | Aircraft’s registration number. |
| Name | The given nomenclature/name of the article (i.e. “Sun Gear”). |
| P/N | Part number of the article. |
| S/N | Serial number of the article. If there is no serial number use “NA”. |
| MPI | X on line indicating if a magnetic particle inspection is required. |
| FPI | X on line indicating if a fluorescent penetrant inspection is required. |

## NDT Inventory Log – FORM MHI 011

Instructions: Non-Destructive Testing (NDT) Inventory Log

This form is an inventory log of articles sent for non-destructive testing. It is initiated by a mechanic. It will be archived with the work order package.

Note: Leave no blanks – use ‘N/A’ if not required or not applicable.

| W/O # | Work order number assigned on form MHI 002 |
| --- | --- |
| A/C Type | Aircraft type (i.e. “Bell 407”). |
| A/C N# | Aircraft’s registration number. |
| A/C S/N | Aircraft’s serial number. |
| A/C T.T. | Aircraft’s total time. |
| Date | Date the form was initiated. |
| Assembly | Higher assembly’s nomenclature, if applicable (i.e. “Main Transmission”). |
| S/N | Serial number of the higher assembly, if applicable. |
| Qty | Number/quantity of pieces for the given article. |
| Nomenclature | Given nomenclature of the article (i.e. “Sun Gear”). |
| Part Number | Enter the part number of the article. |
| Serial Number | Enter the serial number of the article. If there no serial number, use “NSN” (“No Serial Number”). |
| MPI | “X” in the appropriate box if this type of inspection is required for the article. |
| FPI | “X” in the appropriate box if this type of inspection is required for the article. |
| Performed By | For vendor use only. |
| Work Order # | For vendor use only. |
| Accepted By | For vendor use only. |
| Notes | Use this space to note anything unusual or noteworthy. If no notes, enter “N/A”. |
| Received, Inspected By | Individual(s) receiving and inspecting the article must sign/date here. |

## Shelf Life Audit – FORM MHI 012

Instructions: Shelf Life Audit Sheet

This form documents that an audit of shelf life items in the storage unit was performed. It will be initiated by the person responsible for monitoring MHI operated repair station shelf life products. Shelf life audits are to be conducted by the end of each month. This form will be discarded upon initiation of a new, annual form.

| Date | Day the audit is accomplished for the appropriate month. |
| --- | --- |
| Name | Printed name of individual performing the audit. |
| Initials | Individual’s initials. |
| Results | Brief description of results. |

## Inventory Part Receiving Label – FORM MHI 013 (Fairbanks Only)

Instructions: Inventory Part Receiving Label

This form is placed on part after an incoming and receiving inspection has been performed.

| P/N | Manufacturer Part Number |
| --- | --- |
| Description | Nomenclature of Part |
| S/N | Serial Number if one present |
| L/B | Lot or Batch Number |
| Location | What location on the parts shelf |
| Shelf Life Expiration Date | Enter date expires if item has a shelf life |
| Date | Date the incoming/receiving inspection performed |
| Signature | Signature of person performing the incoming/receiving inspection |

## Inventory Part Receiving Label – FORM MHI 014 (Homer Only)

Instructions: Inventory Part Receiving Label

This form is placed on part packaging after an incoming and receiving inspection has been performed.

| P/N | Manufacturer Part Number |
| --- | --- |
| Description | Nomenclature of Part |
| S/N | Serial Number if one present |
| L/B | Lot or Batch Number |
| Location | What location on the parts shelf |
| Expiration | Enter date expires if item has a shelf life |
| Cost | Optional |
| Date | Date the incoming/receiving inspection performed |
| Signature | Signature of person performing the incoming/receiving inspection |

## Summary of Employment – FORM MHI 015

Name: _____________________________________________________________

Scope of Assignment:  Maritime Helicopters Maintenance

Present Title: ________________________________________________________

Years’ experience in this type work: ______________________________________

Past employment record: (at least 5 years)

| From: | To: | Place: | Type of Work | Position |
| --- | --- | --- | --- | --- |

Authorized to sign:   All Inspections & Maintenance per MHI Inspection Procedures Manual

Type and number of FAA Certificate(s) now held:

1.   ____________________________________________________________________________

2.  ____________________________________________________________________________

3.  ____________________________________________________________________________

Instructions: Summary of Employment

This form shall be utilized to capture information required by Title 14 CFR §145.161(a)(4) for management, supervisory, inspection, and return to service personnel. For all other repair station positions, this is optional. This shall be maintained by the Chief Inspector during the individual’s period of employment.

| Employee Name | Name |
| --- | --- |
| Scope | Define scope of present employment. |
| Present Title | Title |
| Years’ Experience | Number of years in this type of work |
| Past Employment | Enter past 5 years of experience |
| Dates of Employ | Dates of previous employment |
| Place | Name of previous employment |
| Type of Work | Types of maintenance work performed (attach add ’l sheets) |
| Type FAA Certification | FAA Certificates currently held (A&P, IA, Repairman, etc.) |
| and Cert Number | Certification Numbers |

## Calibration Sticker (sample) – FORM MHI 016

Instructions: Calibration Sticker

This sticker is used when calibration in performed in-house by Maritime Helicopters. The wording and size of information might vary in order to fit the necessary information on the sticker. This sticker is to remain on the calibrated tool until the tool is recalibrated. The manufacturers calibration sticker shall remain on if performed outside MHI.

In the event the manufacturers sticker becomes to worn to read or detaches, this sticker may be used, transferring the information.  NOTE:  Verify with received calibration paperwork, which is to be kept on record with each recal.

ID Number		Enter I.D. No. of the tool (Either model number or S/N)

By			Initials of person performing the calibration

Date			Enter date of calibration

Date due		Enter tool re-calibration due date

## Capabilities Self Evaluation Checklist – FORM 017

Instructions: Capabilities Self Evaluation Checklist

This form shall be utilized for the purposes of performing a self-evaluation prior to modifying the Repair Station Capabilities List. This self-evaluation shall also be utilized for the purpose of performing an annual review of the capabilities. Attachments may be utilized if needed for additional information.

Description 	Indicate the nature of the Self Evaluation (Add, Delete, modify an existing capability, or perform an Annual Review), Capability to be performed.

Items 1 Thru 5		Answer each question with either a Yes, No or N/A as applicable

Notes	Enter any relevant notes pertaining to the aircraft or article being evaluated.

Signature	Enter signature of the person performing the evaluation.

Date	Enter the Capability Checklist was performed.

## Tool Calibration Record – MHI Form 018

Instructions: Tool Calibration Record

This is used track and record tools that must be calibrated, S/N, owner and due date of next recal.

Group			Enter the application

Description 		Enter the name of the tool

S/N			Enter serial number of the tool

Owner			Enter company or mechanic’s name

C/W			Enter date tool was inspected

Due			Enter tool re-calibration due date

Notes			Enter relevant notes on status of tool

Part			Enter what CFR or Personal the tool belongs to

## Inventory Sign Out Tag – Form MHI 019

## Required Logbook Entries Tag – Form MHI 020

## Mechanic Signature/Initial Roster - Form MHI 021

## Altimeter 24 Month Checklist Form MHI 022

## Transponder 24 Month Checklist Form MHI 023

## Repair Station Inspection and Return to Service Roster - Form MHI 024

| Inspection/Return to Service Roster Homer and Fairbanks Part 145 Repairs Stations CFR 145.161(a)(2,3) MHI 024 |  |  |  |  |
| --- | --- | --- | --- | --- |
| NAME | TITLE | APPROVALS | CERT # | DATE AUTHORIZED |

## Repair Station Management and Supervisory Roster - Form MHI 025