Fueling Facility Monthly Audit

Fueling facilities that are frequented by HFS aircraft shall have a monthly audit using this checklist.  Results of the audit shall be kept on file for one year.  They audit may be performed by either a qualified pilot or mechanic.

Ref: HFS GOM Chapter 5

- Ensure fuel truck has been inspected for contamination and recorded…………………………………….

- Check condition of bonding cables, no frying, breaks and/or excessive corrosion……………………

- Ensure Fueling nozzle has a bonding cable to attach to aircraft………………………………………………..

- Ensure fueling point has been checked that day for contamination and results recorded….…….

- Ensure fueling location has a proper fire extinguisher readily available………………………….………..

- Are there any visible fuel leaks to include hoses and nozzles…………………………………………….……..

- Check for appropriate decals such as No Smoking are present and legible………………………….……

- Check any filters to ensure replacement is not overdue.  A Filter Due Replacement sticker should be readily visible…………………………………………………………………………………………………….………….………

If suspected fuel contamination, contact the Director of Operations or Director of Maintenance for guidance and further testing.

Notes: _________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Printed  Name________________________  Date __________________

Signature ___________________________________________________